#include <stm32f10x_lib.h>                        // STM32F10x Library Definitions

#include "STM32_Init.h"                 /* STM32 Initialization              */

#include "flash.h"

#include "Drive.h"

#include "STM32_Reg.h"

#include "PrinterParType.h"

#include "ParseESC.h"

#include "ReceiveBufferPro.h"

#include "LabelPro.h"

#include "IICTimer.h"

#ifdef PRINTER_AND_USB

#include "usb_lib.h"

#include "hw_config.h"

#endif

//软件延时程序，不是精确的延时

#define U8	unsigned  char

#define U16	unsigned  short

#define U32	unsigned  int

#define CR_OPTER_Set             ((unsigned int)0x00000020)

#define CR_STRT_Set              ((unsigned int)0x00000040)

#define CR_OPTER_Reset           ((unsigned int)0x00001FDF)

#define CR_OPTPG_Set             ((unsigned int)0x00000010)

#define CR_OPTPG_Reset           ((unsigned int)0x00001FEF)

#define RDP_Key                  ((unsigned short)0x00A5)

#define FLASH_KEY1               ((unsigned int)0x45670123)

#define FLASH_KEY2               ((unsigned int)0xCDEF89AB)

#define EraseTimeout          ((unsigned int)0x000B0000)

//*****************************************************************************

//***************计算参数表**************************************************************

const char SelfTestSerial[]={"SN:32F06430081605\r"};

#ifdef PRINTER_PROTOCOL

const char SelfTestD_1[]={"Interface:serial\rProtocol1\r"};

const char SelfTestD_P[]={"Interface:Parallel\rProtocol1\r"};

#endif

#ifndef PRINTER_PROTOCOL

const char SelfTestD_1[]={"Interface:serial\r"};

const char SelfTestD_P[]={"Interface:Parallel\r"};

#endif

const char CompileDate[] = __DATE__;

const char CompileTime[] = __TIME__;

const char SelfTestD_150[]={"serial Baud:150"};

const char SelfTestD_300[]={"serial Baud:300"};

const char SelfTestD_600[]={"serial Baud:600"};

const char SelfTestD_1200[]={"serial Baud:1200"};

const char SelfTestD_2400[]={"serial Baud:2400"};

const char SelfTestD_4800[]={"serial Baud:4800"};

const char SelfTestD_9600[]={"serial Baud:9600"};

const char SelfTestD_19200[]={"serial Baud:19200"};

const char SelfTestD_38400[]={"serial Baud:38400"};

const char SelfTestD_57600[]={"serial Baud:57600"};

const char SelfTestD_115200[]={"serial Baud:115200"};

const char Setenergy[]={"energy consumption:"};

const char Setresort[]={"Restore default Settings\r"};

//8 无校验

const char SelfTestD_8N1[]={",N,8,1\r"};

//8 奇校验

const char SelfTestD_8O1[]={",O,8,1\r"};

//8 偶校验

const char SelfTestD_8E1[]={",E,8,1\r"};

//7 偶校验

const char SelfTestD_7E1[]={",E,7,1\r"};

const char Self_Reverse[]={"Reverse printing\r"};

const char Self_Forward[]={"Forward printing\r"};

const char SelfTestD_Ctrl[]={"flow ctrl:hardware\r"};

const char SelfTestD_XFCtrl[]={"flow ctrl:Xon-Xoff\r"};

const char SelfTestD_2[]={"Installed Fonts:\r\

0:ANK 6X8\r\

1:SYSTEM 12X24\r\

"};

const char SelfTestD_2_1[]={"Installed Fonts:\r\

0:ANK 6X8\r\

1:SYSTEM 8X16\r\

"};

const char SelfTestD_24_GB2312[]={"2:24X24 国标一二级汉字库\r"}	 ;

const char SelfTestD_16_GB2312[]={"2:16X16 国标一二级汉字库\r"}	  ;

const char SelfTestD_24_GBK[]={"2:24X24 GB18030汉字库\r"};

const char SelfTestD_16_GBK[]={"2:16X16 GB18030汉字库\r"};

const char GB2312_24x24[]={"GB2312_24x24"};

const char GB18030_24x24[]={"GB18030_24x24"};

const char GB2312_16x16[]={"GB2312_16x16"};

const U8 SelfTest_S[]={'F','u','l','l',' ','B','a','r','c','o','d','e',' ','S','u','p','p','o','r','t','.',0x0D,

0x1D,0x6B,0x49,14,0x7B,0x41,'4','0','0','0','-','8','5','0','-','8','8','0',0x0D,

0x43,0x68,0x61,0x72,0x61,0x63,0x74,0x65,0x72,0x20,0x53,0x65,0x74,0x3A,0x28,0x6D,0x6F,0x64,0x69,0x66,0x69,0x65,0x64,0x29,0x1C,0x38,0x0D,0xff};

#ifndef NONAME_SELF

const U8 SelfTest_COM[]={"\

Tel:+86-10-62105070\r\

Web:www.rd-cn.com\r\

\r\

\r\

\r\

Selftest Finished!\r\r\rDUMP!\r\r\r"};

#endif

#ifdef NONAME_SELF

const U8 SelfTest_COM[]={"\

\r\

\r\

\r\

Selftest Finished!\r\r\rDUMP!\r\r\r"};

#endif

const U32 CHAR_6X8_1[]={

0x00000000,0x00000000,0x0000FA00,0x00000000,0x00E000E0,0x00000000,0x28FE28FE,0x28000000,0x2454FE54,0x48000000,0xC4C81026,0x46000000,0x6C926A04,0x0A000000,0x0020C080,0x00000000,

0x38448200,0x00000000,0x00008244,0x38000000,0x4428FE28,0x44000000,0x1010FE10,0x10000000,0x00020C08,0x00000000,0x10101010,0x10000000,0x00000606,0x00000000,0x04081020,0x40000000,

0x7C8A92A2,0x7C000000,0x0042FE02,0x00000000,0x468A9292,0x62000000,0x848292B2,0xCC000000,0x182848FE,0x08000000,0xE4A2A2A2,0x9C000000,0x3C529292,0x8C000000,0x808E90A0,0xC0000000,

0x6C929292,0x6C000000,0x62929294,0x78000000,0x00006C6C,0x00000000,0x00026C6C,0x00000000,0x10284482,0x00000000,0x28282828,0x28000000,0x00824428,0x10000000,0x40809AA0,0x40000000,

0x7C82BAAA,0x7A000000,0x3E488848,0x3E000000,0xFE929292,0x6C000000,0x7C828282,0x44000000,0xFE828282,0x7C000000,0xFE929292,0x82000000,0xFE909090,0x80000000,0x7C828A8A,0x4E000000,

0xFE101010,0xFE000000,0x0082FE82,0x00000000,0x040282FC,0x80000000,0xFE102844,0x82000000,0xFE020202,0x02000000,0xFE403040,0xFE000000,0xFE201008,0xFE000000,0x7C828282,0x7C000000,

0xFE909090,0x60000000,0x7C828A84,0x7A000000,0xFE909894,0x62000000,0x64929292,0x4C000000,0x8080FE80,0x80000000,0xFC020202,0xFC000000,0xF8040204,0xF8000000,0xFE041804,0xFE000000,

0xC6281028,0xC6000000,0xC0201E20,0xC0000000,0x868A92A2,0xC2000000,0xFEFE8282,0x00000000,0x40201008,0x04000000,0x008282FE,0xFE000000,0x2040FE40,0x20000000,0x10385410,0x10000000,

0x00804020,0x00000000,0x242A2A1C,0x02000000,0xFE142222,0x1C000000,0x1C222222,0x10000000,0x1C222214,0xFE000000,0x1C2A2A2A,0x10000000,0x107E9090,0x40000000,0x19252525,0x1E000000,

0xFE102020,0x1E000000,0x00009E00,0x00000000,0x01119E00,0x00000000,0xFE081422,0x02000000,0x0082FE02,0x00000000,0x1E201E20,0x1E000000,0x201E2020,0x1E000000,0x1C222222,0x1C000000,

0x3F242424,0x18000000,0x18242424,0x3F000000,0x201E2020,0x10000000,0x122A2A2A,0x24000000,0x20FC2222,0x04000000,0x3C02023C,0x02000000,0x38040204,0x38000000,0x3C023C02,0x3C000000,

0x22140814,0x22000000,0x39050509,0x3E000000,0x22262A32,0x22000000,0x106C8200,0x00000000,0x0000EE00,0x00000000,0x0000826C,0x10000000,0x10201008,0x10000000,0xAA54AA54,0xAA000000,

0x7C828282,0x7C000000,0x10101010,0x10000000,0x04444444,0x04000000,0x82929292,0x82000000,0xFEFA82FA,0xFE000000,0xA2FEA2BE,0x82000000,0x222CE02C,0x22000000,0x20FE2222,0x24000000,

0x02FC00FC,0x02000000,0x22FC203E,0x02000000,0x2020FE20,0x20000000,0x22BCA0BE,0x22000000,0x245CD47E,0x54000000,0x02FCA8AA,0xFE000000,0x007C5454,0x7C000000,0x94543E54,0x94000000,

0x167E9292,0x44000000,0x0059A59A,0x00000000,0x0804FE04,0x08000000,0x10105438,0x10000000,0x20408040,0x20000000,0x2222FA22,0x22000000,0x10105410,0x10000000,0x30483048,0x30000000,

0x24442414,0x24000000,0x08000800,0x08000000,0x0E11110E,0x00000000,0x70888870,0x00000000,0x004898A8,0x48000000,0x009088A8,0xD0000000,0x00091315,0x09000000,0x00121115,0x1A000000,

0x0C120A04,0x3A000000,0x1F245252,0x2C000000,0x3040201E,0x60000000,0x4CB29292,0x4C000000,0x2C525242,0x24000000,0x008472CA,0x84000000,0x403C4040,0x3F000000,0x7C929292,0x7C000000,

0x427C403C,0x02000000,0x7E100870,0x08000000,0x403E0408,0x70000000,0x3A464046,0x3A000000,0xA9555545,0x42000000,0x427C407C,0x82000000,0x023C4848,0x30000000,0x1C222222,0x5C000000,

0x10203C22,0x24000000,0x30493E48,0x30000000,0x30087F08,0x30000000,0x3C421C42,0x3C000000,0xFE808080,0xC0000000,0x040C1424,0x7C000000,0x80FE80FE,0x80000000,0xC6AA9282,0xC6000000,

0xF00AFE0A,0xF0000000,0x7A8E808E,0x7A000000,0xC6BA92BA,0xC6000000,0x7CBA92BA,0x7C000000,0x3E408040,0x3E000000,0x3844FE44,0x38000000,0x40827E82,0x40000000,0x040C1424,0x44000000,

0xFF818181,0x81810000,0x81818181,0x81810000,0xFF818181,0x81FF0000,0x81818181,0x81FF0000,0x80808080,0x80800000,0x01010101,0x01010000,0xFF000000,0x00000000,0x00000000,0x00FF0000,

0x01060810,0x60800000,0x80601008,0x06010000,0xFF808080,0x80800000,0xFF010101,0x01010000,0x01010101,0x01FF0000,0x80808080,0x80FF0000,0x81661818,0x66810000,0xC3E77E7E,0xE7C30000,

0xFFFFC3C3,0xC3C30000,0xC3C3C3C3,0xC3C30000,0xFFFFC3C3,0xFFFF0000,0xC3C3C3C3,0xFFFF0000,0xC0C0C0C0,0xC0C00000,0x03030303,0x03030000,0xFFFF0000,0x00000000,0x00000000,0xFFFF0000,

0x03071E78,0xE0C00000,0xC0E0781E,0x07030000,0xFFFFC0C0,0xC0C00000,0xFFFF0303,0x03030000,0x03030303,0xFFFF0000,0xC0C0C0C0,0xFFFF0000,0x18181818,0x18180000,0x0000FFFF,0x00000000,

0x1818F8F8,0x00000000,0x18181F1F,0x00000000,0x00001F1F,0x18180000,0x0000F8F8,0x18180000,0x1818F8F8,0x18180000,0x18181F1F,0x18180000,0x0000FFFF,0x18180000,0x1818FFFF,0x00000000,

0x1F1F1818,0xF8F80000,0xF8F81818,0x1F1F0000,0x18181C0E,0x07030000,0x03070E1C,0x18180000,0x18183870,0xE0C00000,0xC0E07038,0x18180000,0x18183C7E,0xE7C30000,0xC3E77E3C,0x18180000,

0xF0F0F000,0x00000000,0x0F0F0F00,0x00000000,0x0000000F,0x0F0F0000,0x000000F0,0xF0F00000,0xF0F0F0F0,0xF0F00000,0x0F0F0F0F,0x0F0F0000,0xFFFFFF00,0x00000000,0x000000FF,0xFFFF0000,

0x0F0F0FF0,0xF0F00000,0xF0F0F00F,0x0F0F0000,0xFFFFFFF0,0xF0F00000,0xFFFFFF0F,0x0F0F0000,0x0F0F0FFF,0xFFFF0000,0xF0F0F0FF,0xFFFF0000,0xFFFFFFFF,0xFFFF0000,0x1818FFFF,0x18180000,

};

const U32 CHAR_6X8_2[]={

0xBEAAEAAA,0xBE000000,0x50507E90,0x10000000,0x82FCA0A2,0xBE000000,0x82FE82FE,0x82000000,0xE0BC4242,0x24000000,0xE0A07E50,0x50000000,0x202000F8,0x00000000,0x3050F810,0x00000000,

0x060A121F,0x02000000,0xE8103256,0x8A000000,0xE810226A,0xBE000000,0xE81C347E,0x84000000,0x80E08000,0x00000000,0x44281028,0x44000000,0x04080402,0xFC000000,0x0202FE02,0x02000000,

0x0638C638,0xC0000000,0x00FE00FE,0x00000000,0xFC020202,0xFC000000,0x7E808080,0x7E000000,0x7C92FE92,0x7C000000,0x38444444,0x44000000,0x44444444,0x38000000,0x38549292,0x00000000,

0x3854FE54,0x00000000,0xE0382438,0xE0000000,0x30282428,0x30000000,0x58545454,0x38000000,0x00027C80,0x00000000,0x003A6CB8,0x00000000,0x24247E24,0x18000000,0xC0000C00,0xC0000000,

0x0C00C000,0x0C000000,0x54545454,0x54000000,0xD4345494,0xD4000000,0x30501014,0x18000000,0x282C3868,0x28000000,0x18241824,0x24000000,0x08142A44,0x82000000,0x82442A14,0x08000000,

0x1028FE44,0x00000000,0x0044FE28,0x10000000,0x2046FA46,0x20000000,0x08C8BEC8,0x08000000,0x4444FE44,0x44000000,0x4040FE40,0x40000000,0xC2CC3646,0x86000000,0xC6000000,0xC6000000,

0x54285428,0x54000000,0xB4484848,0xB4000000,0x7C820000,0x00000000,0x00000082,0x7C000000,0x102854AA,0x44000000,0x44AA5428,0x10000000,0xFE82BEA0,0xE0000000,0x0E0AFA82,0xFE000000,

0xFE82BAC6,0x00000000,0x00C6BA82,0xFE000000,0x80402040,0x80000000,0x00400040,0x00000000,0x38448244,0x38000000,0x387C1E7C,0x38000000,0x387CFE7C,0x38000000,0x3852FE52,0x38000000,

0x387AE67A,0x38000000,0x8082BC90,0xE0000000,0x08103E40,0x80000000,0x7040C244,0x78000000,0x42427C42,0x42000000,0x444850FE,0x40000000,0x42FC4042,0x7C000000,0x50F05854,0x52000000,

0x30C0427C,0x00000000,0x30C0427C,0x40000000,0x42424242,0x7E000000,0x40F244F8,0x40000000,0xA2520408,0x10000000,0x42444854,0x62000000,0x40FC4252,0x62000000,0x60120204,0x78000000,

0x10E2524C,0x78000000,0x50527C90,0x10000000,0x70007204,0x78000000,0x20A2BCA0,0x20000000,0x00FE1008,0x00000000,0x4244F840,0x40000000,0x02424242,0x02000000,0x42544854,0x60000000,

0x4448DE68,0x44000000,0x00020CF0,0x00000000,0x0438007C,0x02000000,0x04FC1262,0x0E000000,0x40424244,0x78000000,0x20402010,0x08000000,0x4C40FE40,0x4C000000,0x4048444A,0x70000000,

0xA8542A00,0x00000000,0x1C244404,0x0E000000,0x02140814,0x60000000,0x507C5252,0x52000000,0x20FE2028,0x30000000,0x0242427E,0x02000000,0x52525252,0x7E000000,0x20A0A2A4,0x38000000,

0x00F00204,0xF8000000,0x3E007E02,0x1C000000,0x7E020408,0x10000000,0x7E424242,0x7E000000,0x70404244,0x78000000,0x087848FE,0x48000000,0x02425E52,0x62000000,0xA2A2A4A8,0xF0000000,

0x42220408,0x10000000,0x20222C28,0x30000000,0x18103212,0x1C000000,0x12121E12,0x12000000,0x1214183E,0x10000000,0x103E1014,0x18000000,0x0212121E,0x02000000,0x2A2A2A3E,0x00000000,

0x18021204,0x18000000,0x40209040,0x20000000,0x40A0A040,0x00000000,0xFE92928C,0xC0000000,0x8EF48484,0xFE000000,0x7ED252D2,0x44000000,0xC638FE38,0xC6000000,0xCC829292,0x6C000000,

0x7E041820,0x7E000000,0x3E844890,0x3E000000,0x82FC80FE,0x82000000,0x84FC04FC,0x86000000,0xF0080810,0xFE000000,0xFC04FC04,0xFC000000,0xFC04FC04,0xFE000000,0x80FE1212,0x0C000000,

0xFE12120C,0xFE000000,0xC48292D6,0x7C000000,0xFE107C82,0x7C000000,0x62949890,0xFE000000,0x3C525252,0x8C000000,0x50AAAAAA,0x14000000,0x1CAA6A2A,0x18000000,0x38467CC4,0x38000000,

0x18263C64,0x18000000,0x708A8A8C,0x50000000,0xBC0202BC,0x02000000,0x1C2A6AAA,0x10000000,0x44AAAA9C,0x42000000,0x04AA2A9C,0x02000000,0x04AA6A1C,0x02000000,0x042AAA1C,0x02000000,

0x304A4A4C,0x20000000,0x5CAAAAAA,0x58000000,0x1CAA2AAA,0x18000000,0x1C2AAA2A,0x18000000,0xA23E8200,0x00000000,0x40929E82,0x40000000,0x925E0200,0x00000000,0x0E942494,0x0E000000,

0x0E54A454,0x0E000000,0x3E2A6AAA,0x22000000,0x2C2A3C2A,0x1A000000,0x3E50FE92,0x92000000,0x4C92924C,0x00000000,0x9C22229C,0x00000000,0x1CA2621C,0x00000000,0x5C82825C,0x02000000,

0x3C82423C,0x02000000,0x20920C90,0x20000000,0x1CA222A2,0x1C000000,0x3C820282,0x3C000000,0x18247E24,0x24000000,0xFEA0E81E,0x0A000000,0x127E9080,0x40000000,0x042A6A9C,0x02000000,

0x00125E82,0x00000000,0x1C22629C,0x00000000,0x3C42823C,0x02000000,0x409E905E,0x80000000,0x5E888442,0x9E000000,0x12AAAA72,0x0A000000,0x728A8A72,0x00000000,0x0C12A202,0x04000000,

0x3A4A8C88,0x50000000,0x0C32820C,0xB2000000,0x0C1A6AAA,0x14000000,0x044AAAAC,0x52000000,0x040AAA2C,0x92000000,0x040AAA6C,0x12000000,0x040A2A2C,0x92000000,0x1A2A4C48,0x20000000,

0x0C5AAAAA,0x50000000,0x0C1AAA2A,0x90000000,0x0C1AAA6A,0x10000000,0x0222AE32,0x80000000,0x02569A80,0x40000000,0x02965A00,0x00000000,0x06089828,0x9E000000,0x060858A8,0x5E000000,

0x061A2A6A,0xA0000000,0x042A3E2A,0x10000000,0x0E305EF2,0x82000000,0x044A9294,0x48000000,0x0C12A224,0x98000000,0x0C12A264,0x18000000,0x0C52828E,0x52000000,0x0C32824E,0x32000000,

0x328C0810,0xA0000000,0x0C12A224,0x98000000,0x0C32820C,0xB0000000,0x0C162C74,0x20000000,0x1E68AEBA,0xE8000000,0x121E7090,0xC0000000,0x042A6AAE,0x1A000000,0x02165A80,0x00000000,

0x0C122264,0x98000000,0x0C32024E,0xB2000000,0x528C9052,0x8C000000,0x46988846,0x9C000000,0x122AAABA,0x6A000000,0x324A8A92,0x62000000,0x0C122282,0x04000000,0x1C202020,0x1C000000,

};

const unsigned char chInvTable16[] = { 0x00, 0x08, 0x04, 0x0C,

0x02, 0x0A, 0x06, 0x0E,

0x01, 0x09, 0x05, 0x0D,

0x03, 0x0B, 0x07, 0x0F };

const U32 DiV_config[9]={0,65536,32768,21846,16384,13108,10923,9363,8192};

const U32 Config_zoom[128]={

0x00000000,0x00000001,0x00000002,0x00000003,0x00000004,0x00000005,0x00000006,0x00000007,0x00000008,0x00000009,0x0000000A,0x0000000B,0x0000000C,0x0000000D,0x0000000E,0x0000000F,

0x00000000,0x00000003,0x0000000C,0x0000000F,0x00000030,0x00000033,0x0000003C,0x0000003F,0x000000C0,0x000000C3,0x000000CC,0x000000CF,0x000000F0,0x000000F3,0x000000FC,0x000000FF,

0x00000000,0x00000007,0x00000038,0x0000003F,0x000001C0,0x000001C7,0x000001F8,0x000001FF,0x00000E00,0x00000E07,0x00000E38,0x00000E3F,0x00000FC0,0x00000FC7,0x00000FF8,0x00000FFF,

0x00000000,0x0000000F,0x000000F0,0x000000FF,0x00000F00,0x00000F0F,0x00000FF0,0x00000FFF,0x0000F000,0x0000F00F,0x0000F0F0,0x0000F0FF,0x0000FF00,0x0000FF0F,0x0000FFF0,0x0000FFFF,

0x00000000,0x0000001F,0x000003E0,0x000003FF,0x00007C00,0x00007C1F,0x00007FE0,0x00007FFF,0x000F8000,0x000F801F,0x000F83E0,0x000F83FF,0x000FFC00,0x000FFC1F,0x000FFFE0,0x000FFFFF,

0x00000000,0x0000003F,0x00000FC0,0x00000FFF,0x0003F000,0x0003F03F,0x0003FFC0,0x0003FFFF,0x00FC0000,0x00FC003F,0x00FC0FC0,0x00FC0FFF,0x00FFF000,0x00FFF03F,0x00FFFFC0,0x00FFFFFF,

0x00000000,0x0000007F,0x00003F80,0x00003FFF,0x001FC000,0x001FC07F,0x001FFF80,0x001FFFFF,0x0FE00000,0x0FE0007F,0x0FE03F80,0x0FE03FFF,0x0FFFC000,0x0FFFC07F,0x0FFFFF80,0x0FFFFFFF,

0x00000000,0x000000FF,0x0000FF00,0x0000FFFF,0x00FF0000,0x00FF00FF,0x00FFFF00,0x00FFFFFF,0xFF000000,0xFF0000FF,0xFF00FF00,0xFF00FFFF,0xFFFF0000,0xFFFF00FF,0xFFFFFF00,0xFFFFFFFF,

};

//***************电机参数加热参数**************************************************************

const U32 PrintMotoHalfStep[8]={MOTO_RUN_HALF_1_CODR,MOTO_RUN_HALF_2_CODR,MOTO_RUN_HALF_3_CODR,MOTO_RUN_HALF_4_CODR,MOTO_RUN_HALF_5_CODR,MOTO_RUN_HALF_6_CODR,MOTO_RUN_HALF_7_CODR,MOTO_RUN_HALF_8_CODR};

const unsigned shor VolRUNTime[]=  {4500,4000,3500,3000,2500,2000,1500,1431,1362,1293,1224,1155,1086,1017,948,879,810,741,672,672,672,672};

const unsigned short VolTHSTB[]=    {4500,4000,3500,3000,2500,2000,1500,1400,1300,1250,1190,1100, 932, 934,846,750,570,530,512,512,512,512};

const unsigned short VolRUNPWM[]=   { 304, 304, 304, 304, 304, 304, 304, 304, 304, 304, 304, 304, 304, 304,304,304,304,304,304,304,304,304};

const unsigned short VolFreeTime[]= {1000,1000,1000,1000,1000,1000,1000, 950, 900, 850, 800, 750, 700, 650,600,550,500,450,400,400,400,400};

const unsigned short VolTemAdd[]= 	{ 300, 300, 300, 300, 300, 250, 200, 192, 184, 176, 166, 158, 150, 142,134,126,118,108,100,100,100,100};

const unsigned short VolTemSubb[]= 	{ 200, 200, 200, 200, 200, 150, 100,  96,  92,  88,  84,  80,  76,  72, 68, 64, 60, 56, 50, 50, 50, 50};

//************全局变量*****************************************************************

extern unsigned int   Getdata_ok;//判断ISP数据是否接收完成

#ifdef AUTO_FEED

extern	unsigned short g_printChar;

#endif

//------------数据缓冲区------------------------------------------------------------------------

unsigned char  receive_bufer[1024*2]={0};

extern unsigned short Receive_count;

extern unsigned int	g_Receive_count;//数据接收

extern unsigned int	g_Order_count;//取命令计数器

extern unsigned int *p_OcAndGDAdr;//得到数据的指针

extern  unsigned short *p_REAndSAcount;//保存数据的指针

extern unsigned int *p_SaveDrREcount;//存储数据到缓冲区的偏移量

//------------------------------------------------------------------------------------

unsigned short	g_CalcPrintPos;//打印数据当前的位置

unsigned short	g_PrintLineCount;//打印行数计数器

unsigned short	g_MotoAll;//电机走步索引

//unsigned short	g_BufferLength;//接受数据长度

//unsigned short	g_MotoSTBStatus;//加热使能标志

unsigned short	g_BufferSwitch;//链表缓冲区切换标志

#define DATA_IN_SEND				0x5A67

#define DATA_SEND_OK				0x57A9

unsigned short	g_SendFlag;//链表缓冲区切换标志

unsigned short	g_mapCount;//图形数据计数器

unsigned char 	CodeBarStr[40];

unsigned char	HTValue[20];//保存水平造表值

unsigned char	HTCount;//水平造表计数器

unsigned short	g_0D0AFlag;//0D0A打印标志

extern unsigned char	g_0D0AHigh;//回车走纸高度

//-----------------------------------------------------------------------------

unsigned short	g_PrintStatusReg;//命令链表解析完成状态

//****************************************************************************

GetOrderPoint	s_PrintOrderstr;//取命令结构

PrinterStruct	s_PrintParameters;//打印机参数

PrintControl	s_PrinterMOTOPar;//打印机步进电机参数

CodeBarCon		s_CodeBarCon;//条码控制参数

PrintrConfig	s_PrintrConfig;//打印机配置参数

unsigned int	m_OrderSave;//保留取数指针

//	unsigned short	m_flag;//

extern unsigned int   Getdata_ok;//判断ISP数据是否接收完成

PrintListPar		s_ListParmeter_A;//缓冲区A

PrintListPar		s_ListParmeter_B;//缓冲区B

unsigned short	Buffer_Dataflash[36];

unsigned short	Buffer_CodeBar[BUFFER_SHORT_LENGTH];//条码缓冲区

unsigned short	g_alignType;//左右中 对齐方式

unsigned short	g_SpeedChoice;//设置打印机运行速度

unsigned short	g_DriverAFlag;//驱动打印标志

//	unsigned short	g_CutterErr;//

//	unsigned int	g_BUSYFlag;//Busy 标志

extern unsigned short*	Buffer_writer_I;//[10];//条码缓冲区 可以公用

p_PrintListPar			p_ListParmeters	;//链表结构指针

//int dd;

unsigned short*	p_ListDataDot;//链表中字符图形的存放位置

unsigned int FlowFlag;//波特率

unsigned short	g_dotSwitch;//预留点数切换

unsigned int 	print_head_length;//			384

unsigned short g_StandbyPar;

unsigned short	m_charType;//字符类型

unsigned short*	p_MapCurListDataDot;//图形曲线指针位置

extern unsigned short g_CurveCountFlag;

unsigned short	m_high;//字符高

unsigned short	m_Wide;//字符宽

unsigned short	m_CharWide;//字符宽

unsigned short	m_SaverightPos;//右宽度

unsigned short	m_UPDown;//上下对齐

unsigned short	m_len;//点阵数据长度

unsigned short	g_SavePrintPos;//保存左限

unsigned short*	p_ListSave;//链表保存指针

unsigned short	m_GetData;

p_PrintListPar	p_ListPrease;//链表结构指针

unsigned short*	p_IndexListData;//链表单元索引地址

unsigned short	m_ProcessFlag;//命令处理标志

unsigned int	m_ProcessStatus;

unsigned int	g_THDeep;//增加加热值

unsigned short	g_PaperEnd;//缺纸

//****************************************************************************

extern unsigned short AD_Value[3];

/****************************************************************************/

unsigned short g_TimeOutFlag;

unsigned int			g_lineBitCount,g_S_lineBitCount;//计算点行中加热点数

unsigned short			g_RunFlag;//加热运行标志

unsigned short			SendBufferToSPI[BUFFER_SHORT_LENGTH];

unsigned short			SaveToBuffer[BUFFER_SHORT_LENGTH];

unsigned int			SendBuffer_counter;

unsigned int			SendToSPI_counter;

unsigned short			g_LPSStatus,g_PE_Standard;//缺纸检测器状态

unsigned short			g_RunLTime;

extern  unsigned short g_CodePage;//国际字符代码页

extern unsigned short g_InternationChar;//国际字符集

//****************************************************************************

extern unsigned short g_ConfigRCC;

extern unsigned short	g_unicodeType;//unicode计数器

extern  unsigned int	Erase_Sector;

extern  unsigned int	Order_Erase_Sector;

extern unsigned int		g_SaveDataToFlashAddr;//存储数据的当前地址

extern unsigned int		g_GetDataFromFlashAddr;//从flash中取数据的地址

extern unsigned int		g_SaveDataOffset;//存储数据的当前偏移量

extern unsigned int		g_GetDataOffset;//得到储数据的当前偏移量

extern unsigned short	g_InterFaceBufferCount;//端口接收数据缓冲区接收到的数据个数

extern unsigned short	g_ThType;//加热类型

extern unsigned short	g_cruve_arry[20];

extern unsigned short	g_cruvefalg;

extern unsigned short	g_PrinterDip;//打印DIP

extern unsigned char	g_HTCharSpace;//水平造表计算值

unsigned short			g_ParTypeValue;//分区类型值

unsigned short* p_PrCon;//打印机配置指针

extern unsigned short g_ListControlFor;//字符控制字格式

extern unsigned short g_ListPrintFinish;//是否打印完成

extern unsigned short g_PaperRunToOut;//出纸步数

extern unsigned char g_ListSaveToFlashCount;//数据存储到FLash位置偏移量计数

extern unsigned short	g_PrintListOrNorm;//是否是打印表格或者正常数据

void SendParData(unsigned short u16_anddata);

void SendDICLK(unsigned short u16_data);

void  PrinterParameterInt(void);

void SetSelftestData(void);

void PrinterConfigInt(void);

void SavePrinterConfig(void);

void ConfigToPrinter(void);//

void SetPrtintDataBaud(unsigned char baudvalue);

void SetPrtintDataParity(unsigned char baudvalue);

void SetPrtintDir(unsigned char baudvalue);

void ProKeySetModel(void);

void SetDataBaud(unsigned char baudvalue);

void SetSerialContrl(unsigned char baudvalue);

void DefaultSet(void);

void SetSoftwarePart(unsigned char baudvalue);

void SetPrintFont(unsigned char baudvalue);

#ifdef PRINTER_AND_USB

extern void returnstatus(unsigned char flag);

#endif

extern unsigned short g_ListTable[BUFFER_SHORT_LENGTH];//

extern unsigned char(*PrinterCore)(void);//外部定义的函数指针

extern unsigned short	g_receiveFlage;

unsigned short	g_DumpReceiveFlage;

unsigned short g_ProFuncValue;

extern unsigned short g_PE_UP_Count;//曲线下降趋势计数器

extern unsigned short g_PE_DN_Count;//曲线上升趋势计数器

extern unsigned short g_PEDataS;//采样值预存器

extern unsigned short key_Cont;

extern unsigned short g_PE_Staue;//整型后波形当前状态

extern unsigned char * p_LabData;

unsigned char g_SelfRunFlag;//自检运行标志

unsigned char Trg,Cont;

void (*StandardSetPro[]) (unsigned char dat) =

{

SetPrtintDataBaud,

SetPrtintDataBaud,

SetPrtintDataParity,

SetSerialContrl,

SetPrtintDir,

SetSoftwarePart,

SetPrintFont,

};

void ProTraehdat(unsigned short rule)

{

if(rule == 0)

{

//配置数据到数组

//配置波特率

g_cruve_arry[1] = s_PrintrConfig.m_BaudRate &  0x0F;

if((s_PrintrConfig.m_BaudRate&0x30) == 0x00)

{

//无校验

g_cruve_arry[2] = 0;

}

if((s_PrintrConfig.m_BaudRate&0x30) == 0x20)

{	//Odd 奇效验

g_cruve_arry[2] = 1;

}

if((s_PrintrConfig.m_BaudRate&0x30) == 0x10)

{	//Even 偶效验

g_cruve_arry[2] = 2;

}

if((s_PrintrConfig.m_BaudRate&0x30) == 0x30)

{//7位偶效验

g_cruve_arry[2] = 3;

}

if((s_PrintrConfig.m_BaudRate & 0x40) == 0)

{

g_cruve_arry[3] = 0;//硬件流控

}

else

{

g_cruve_arry[3] = 1;//软件流控

}

if(s_PrintrConfig.m_PrintDir == PRINT_DIRECT_NOR)

{

g_cruve_arry[4] = 0;//正向为0

}

else

{

g_cruve_arry[4] = 1;//反向为1

}

if(s_PrintrConfig.m_SoftwarePart < 13)

{

s_PrintrConfig.m_SoftwarePart &= 0xFE;

g_cruve_arry[5] = (s_PrintrConfig.m_SoftwarePart/2)-1;

}

else

{

g_cruve_arry[5] = 6;

}

if(s_PrintrConfig.m_ChineseFont == PRINTTYPE_24)

{

if(s_PrintrConfig.m_ChineseLib == TYPE_GBK)

{

//

g_cruve_arry[6] = 1;

}

else

{

g_cruve_arry[6] = 0;

}

}

else

{

g_cruve_arry[6] = 2;

}

}

else

{

//数组到配置数据

s_PrintrConfig.m_BaudRate = g_cruve_arry[1];

if(g_cruve_arry[2] == 0x00)

{

//无校验

s_PrintrConfig.m_BaudRate &= 0xCF;

}

if(g_cruve_arry[2] == 0x01)

{	//Odd 奇效验

s_PrintrConfig.m_BaudRate &= 0xCF;

s_PrintrConfig.m_BaudRate |= 0x20;

}

if(g_cruve_arry[2] == 0x2)

{	//Even 偶效验

s_PrintrConfig.m_BaudRate &= 0xCF;

s_PrintrConfig.m_BaudRate |= 0x20;

}

if(g_cruve_arry[2] == 3)

{//7位偶效验

s_PrintrConfig.m_BaudRate |= 0x30;

}

if(g_cruve_arry[3] == 0)

{

//硬件流控

s_PrintrConfig.m_BaudRate &= 0x3F;

}

else

{

s_PrintrConfig.m_BaudRate |= 0x40;//软件流控

}

if(g_cruve_arry[4] == 0)

{

//正向为0

s_PrintrConfig.m_PrintDir = PRINT_DIRECT_NOR;

}

else

{

//反向为1

s_PrintrConfig.m_PrintDir = PRINT_DIRECT_FX;

}

if(g_cruve_arry[5] < 6)

{

s_PrintrConfig.m_SoftwarePart = (g_cruve_arry[5]+ 1)*2;

}

else

{

s_PrintrConfig.m_SoftwarePart = 16;

}

if(g_cruve_arry[6] < 2)

{

if(g_cruve_arry[6] == 0)

{

s_PrintrConfig.m_ChineseLib = TYPE_GB2312;//默认国标一二级

}

else

{

s_PrintrConfig.m_ChineseLib = TYPE_GBK;//默认国标一二级

}

s_PrintrConfig.m_ChineseFont = PRINTTYPE_24;//默认24点阵机型

s_PrintrConfig.m_DefaultChar = TYPE_ASC12X24;//默认打印字符

s_PrintrConfig.m_DefaultChinese = TYPE_CHIN24X24;//默认打印汉字

s_PrintrConfig.m_HoriZoomOut = 1;//默认横向放大倍数

s_PrintrConfig.m_VertZoomOut = 1;//默认纵向放大倍数

}

if(g_cruve_arry[6] == 2)

{

s_PrintrConfig.m_ChineseLib = TYPE_GB2312;//默认国标一二级

s_PrintrConfig.m_ChineseFont = PRINTTYPE_16;//默认24点阵机型

s_PrintrConfig.m_DefaultChar = TYPE_ASC8X16;//默认打印字符

s_PrintrConfig.m_DefaultChinese = TYPE_CHIN16X16;//默认打印汉字

s_PrintrConfig.m_HoriZoomOut = 2;//默认横向放大倍数

s_PrintrConfig.m_VertZoomOut = 2;//默认纵向放大倍数

}

}

}

unsigned short gTimer_1ms,SysTickFlag;

void SysTick_Handler(void)

{

if(SysTickFlag == 0xAAAA)

{

gTimer_1ms++;

if(gTimer_1ms > 600)

{

g_TimeOutFlag = 0xaa;

}

}

} // end SysTick_Handler

//第一项表示总共的项目数

//const unsigned char ParSetlist[]={2,4,6,8,10,12,16};

const unsigned char ItemIndexCount[]={6,11,4,2,2,7,3};

unsigned short m_ProKeyValue;

void ProKeySetModel(void)

{

unsigned short m_itemMax,i,j;

if((ReadKeyPlus() & 1) == 1)

{

m_ProKeyValue++;

__SETPRIMASK();

SysTickFlag = 0xAAAA;

gTimer_1ms = 0;

g_TimeOutFlag = 0;

__RESETPRIMASK();

//	TIM_DISABLE(TIM6);

//	TIM_ENABLE(TIM6,6000);//演示400ms

}

if(g_TimeOutFlag != 0)

{

//SendChar(m_ProKeyValue);

g_TimeOutFlag = 0;//清定时标志

SysTickFlag = 0;

if(g_ProFuncValue == 0)

{

s_PrintParameters.m_HoriZoomOut = 2;//横向放大倍数为1

s_PrintParameters.m_VertZoomOut = 2;//纵向打印放大为1

s_PrintParameters.m_CharType = TYPE_LITTLECHAR;//默认为0表示打印的是汉字,小于80的表示打印的是字符

s_PrintParameters.m_Charlist = 1;//默认打印字符集1

ProTraehdat(0);

//第一次进入判断按键值是否大于等于9,是表示进入设置状态

if(m_ProKeyValue > 9)

{

g_ProFuncValue = 0x0100 ;//第一项默认是波特率

#ifdef RD_TYPE_PARALLEL

g_ProFuncValue = 0x0400 ;//第一项默认是波特率

#endif

i = (g_ProFuncValue >> 8) & 0xFF;//得到项目值

g_ProFuncValue |= g_cruve_arry[i];

m_ProKeyValue = 0;

}

else

{

m_ProKeyValue = 0;

return ;

}

}

if(m_ProKeyValue == 1)

{

g_ProFuncValue++;

i = (g_ProFuncValue >> 8) & 0xFF;//得到项目值

m_itemMax = ItemIndexCount[i];

j =  g_ProFuncValue & 0xFF;

if(j >= m_itemMax)

{

g_ProFuncValue &= 0xFF00;

}

g_cruve_arry[i] = g_ProFuncValue & 0xFF;

}

if(m_ProKeyValue == 2)

{

g_ProFuncValue += 0x0100;//项目累加

i = (g_ProFuncValue >> 8) & 0xFF;//得到项目值

if(i> ItemIndexCount[0])

{

g_ProFuncValue = 0x0100;//大于最大项会到第一项

#ifdef RD_TYPE_PARALLEL

g_ProFuncValue = 0x0400 ;//第一项默认是波特率

#endif

}

g_ProFuncValue &= 0xFF00;

i = (g_ProFuncValue >> 8) & 0xFF;//得到项目值

g_ProFuncValue |= g_cruve_arry[i];

}

if(m_ProKeyValue > 5)

{

//回复默认值

PrinterConfigInt();

SavePrinterConfig();

DefaultSet();

g_ProFuncValue = 0;

}

if(m_ProKeyValue < 3)

{

i = (g_ProFuncValue >> 8) & 0xFF;//得到项目值

j = g_ProFuncValue & 0xFF;

ProTraehdat(1);

SavePrinterConfig();

(*StandardSetPro[i])(j);

}

m_ProKeyValue = 0;

}

}

/****************************************************************************

* 函数名：PrinterConfigInt()

* 功  能：打印机参数初始化

*

* 说  明：

****************************************************************************/

void PrinterConfigInt(void)

{

s_PrintrConfig.m_InterFace = 0;//软件固定接口方式

s_PrintrConfig.m_CharSpace = 0;//字间距默认为0

s_PrintrConfig.m_LineSpace = 0;//默认行间距0

#ifdef PRINTER_GB2312

s_PrintrConfig.m_ChineseLib = TYPE_GB2312;//默认国标一二级

#endif

#ifdef PRINTER_GBK

s_PrintrConfig.m_ChineseLib = TYPE_GBK;//默认GBK

#endif

#ifdef PRINTER_CHINESE_24

s_PrintrConfig.m_ChineseFont = PRINTTYPE_24;//默认24点阵机型

s_PrintrConfig.m_DefaultChar = TYPE_ASC12X24;//默认打印字符

s_PrintrConfig.m_DefaultChinese = TYPE_CHIN24X24;//默认打印汉字

#endif

#ifdef PRINTER_CHINESE_16

s_PrintrConfig.m_ChineseFont = PRINTTYPE_16;//默认24点阵机型

s_PrintrConfig.m_DefaultChar = TYPE_ASC8X16;//默认打印字符

s_PrintrConfig.m_DefaultChinese = TYPE_CHIN16X16;//默认打印汉字

#endif

s_PrintrConfig.m_PrintDir = PRINT_DIRECT_FX;//打印机打印方向 台式 面板

#ifdef PRINTER_DIRECT_NOR

s_PrintrConfig.m_PrintDir = PRINT_DIRECT_NOR;//打印机打印方向 台式 面板

#endif

s_PrintrConfig.m_HoriZoomOut = 1;//默认横向放大倍数

s_PrintrConfig.m_VertZoomOut = 1;//默认纵向放大倍数

g_0D0AHigh = 24;//

g_HTCharSpace = 12;//根据打印的字符确定空格宽度

if(s_PrintrConfig.m_ChineseFont == PRINTTYPE_16)

{

s_PrintrConfig.m_HoriZoomOut = 2;//默认横向放大倍数

s_PrintrConfig.m_VertZoomOut = 2;//默认纵向放大倍数

g_0D0AHigh = 16;//

}

#ifdef	PRINT_TYPE_48

s_PrintrConfig.m_HoriZoomOut = 1;//默认横向放大倍数

s_PrintrConfig.m_VertZoomOut = 2;//默认纵向放大倍数

#endif

s_PrintrConfig.m_Liftpos = 0;//默认打印机左限

s_PrintrConfig.m_Righttpos = PRINT_HEAD_LENGTH;//默认打印机右限

s_PrintrConfig.m_BaudRate = 0;//串口机型波特率

s_PrintrConfig.m_BaudRate = 6;//串口机型波特率

#ifdef PRINT_BPS_150

s_PrintrConfig.m_BaudRate = 0x0;

#endif

#ifdef PRINT_BPS_300

s_PrintrConfig.m_BaudRate = 0x1;

#endif

#ifdef PRINT_BPS_600

s_PrintrConfig.m_BaudRate = 0x2;

#endif

#ifdef PRINT_BPS_1200

s_PrintrConfig.m_BaudRate = 0x3;

#endif

#ifdef PRINT_BPS_2400

s_PrintrConfig.m_BaudRate = 0x4;

#endif

#ifdef PRINT_BPS_4800

s_PrintrConfig.m_BaudRate = 0x5;

#endif

#ifdef PRINT_BPS_9600

s_PrintrConfig.m_BaudRate = 0x6;

#endif

#ifdef PRINT_BPS_19200

s_PrintrConfig.m_BaudRate = 0x7;

#endif

#ifdef PRINT_BPS_38400

s_PrintrConfig.m_BaudRate = 0x8;

#endif

#ifdef PRINT_BPS_57600

s_PrintrConfig.m_BaudRate = 0x9;

#endif

#ifdef PRINT_BPS_115200

s_PrintrConfig.m_BaudRate = 0x0A;

#endif

#ifdef PRINT_BPS_XONXOFF

s_PrintrConfig.m_BaudRate |= 0x40;

#endif

//Odd 奇效验

#ifdef PRINT_SERIAL_ODD

s_PrintrConfig.m_BaudRate |= 0x20;

#endif

//Even 偶效验

#ifdef PRINT_SERIAL_EVEN

s_PrintrConfig.m_BaudRate |= 0x10;

#endif

//7位偶效验

#ifdef PRINT_SERIAL_7_EVEN

s_PrintrConfig.m_BaudRate |= 0x30;

#endif

s_PrintrConfig.m_SoftwarePart = 4;//软件最大分区

#ifdef PRINTER_LOW_16

s_PrintrConfig.m_SoftwarePart = 16;//软件最大分区

#endif

#ifdef PRINTER_LOW_12

s_PrintrConfig.m_SoftwarePart = 12;//软件最大分区

#endif

#ifdef PRINTER_LOW_10

s_PrintrConfig.m_SoftwarePart = 10;//软件最大分区

#endif

#ifdef PRINTER_LOW_8

s_PrintrConfig.m_SoftwarePart = 8;//软件最大分区

#endif

#ifdef PRINTER_LOW_6

s_PrintrConfig.m_SoftwarePart = 6;//软件最大分区

#endif

#ifdef PRINTER_LOW_4

s_PrintrConfig.m_SoftwarePart = 4;//软件最大分区

#endif

#ifdef PRINTER_LOW_2

s_PrintrConfig.m_SoftwarePart = 2;//软件最大分区

#endif

s_PrintrConfig.m_HotDeep = 0;//额外附加加热深度

s_PrintrConfig.m_PrintSub = 3;//3;//打印机细分度

s_PrintrConfig.m_PrintLabFlage = LABEL_PR_DIS;//设置为0表示无标签打印

s_PrintrConfig.m_LabelWide = 50;//表示打印的纸宽,要根据普通纸58mm纸宽打印宽度48mm 58-50 = 8mm * 8 空64点

s_PrintrConfig.m_LabelLength = 30 * 32;//30mm * 32 1mm=32步

s_PrintrConfig.m_BarPaGap = 3 * 32;////3mm * 32 1mm=32步

//s_PrintrConfig.m_PE_LabValue_L 纸缝判断值 根据自检测试自动计算

//s_PrintrConfig.m_PE_LabValue_H

#ifdef PRINTER_LABEL

s_PrintrConfig.m_PrintLabFlage = LABEL_PR_EN;//设置为标签打印

#endif

s_PrintrConfig.m_HotLineTOPD = 8 * 32; //加热线到传感器的位置

s_PrintrConfig.m_HotLineTOOUT = 5 * 32;//加热线到出纸口的位置

#ifdef RD_TH

s_PrintrConfig.m_HotLineTOOUT = 20 * 32;//加热线到出纸口的位置

#endif

#ifdef PRINTER_BUFF_RAM

s_PrintrConfig.m_ReceiveBuffrWor = RECEIVE_RAM;//RECEIVE_FLASH;

#endif

#ifdef PRINTER_BUFF_FLASH

s_PrintrConfig.m_ReceiveBuffrWor = RECEIVE_FLASH;//RECEIVE_FLASH;

#endif

s_PrintrConfig.m_CharType = TYPE_CHINESE;//默认中文机型

#ifdef PRINTER_ENGLISH

s_PrintrConfig.m_CharType = TYPE_ENGLISH;//默认英文机型

#endif

s_PrintrConfig.m_PrintEndFeed = 0;//默认不走纸

/****************************************************************************

* 函数名：SavePrinterConfig()

* 功  能：打印机参数初始化

*

* 说  明：

****************************************************************************/

void SavePrinterConfig(void)

{

short i;

Buffer_writer_I = &(s_ListParmeter_B.m_ListBuffer[700]);//设置指针位置

p_PrCon = &s_PrintrConfig.m_InterFace;

for(i=0;i<128;i++)

{

Buffer_writer_I[i] = 0;

}

Buffer_writer_I[0] = CompileTime[0] << 8;

Buffer_writer_I[0] |= CompileTime[1];

Buffer_writer_I[1] = CompileTime[3] << 8;

Buffer_writer_I[1] |= CompileTime[4];

for(i=0;i< CONFIG_PAR_LENGTH;i++)

{

Buffer_writer_I[i+2] = p_PrCon[i];

}

ERASE_4K(DL_PRINTER_PAR_ADDR);

AT91F_DataFlashPageWrite_NE(DL_PRINTER_PAR_ADDR,Buffer_writer_I);//写入数据

}

/****************************************************************************

* 函数名：ConfigToPrinter()

* 功  能：配置打印机

*

* 说  明：

****************************************************************************/

void ConfigToPrinter(void)

{

//s_PrintrConfig.m_InterFace = 0;//软件固定接口方式

s_PrintParameters.m_CharSpace =	s_PrintrConfig.m_CharSpace;//字间距默认为0

//s_PrintrConfig.m_ChineseLib = TYPE_GB2312;//默认国标一二级

//s_PrintrConfig.m_ChineseFont = PRINTTYPE_24;//默认24点阵机型

s_PrintParameters.m_CharRepertoire = s_PrintrConfig.m_DefaultChar;// = TYPE_ASC12X24;//默认打印字符

s_PrintParameters.m_ChineseCharType = s_PrintrConfig.m_DefaultChinese;// = TYPE_CHIN24X24;//默认打印汉字

s_PrintParameters.m_PrintDir = s_PrintrConfig.m_PrintDir;// = PRINT_DIRECT_NOR;//打印机打印方向 台式 面板

s_PrintParameters.m_HoriZoomOut = s_PrintrConfig.m_HoriZoomOut;// = 1;//默认横向放大倍数

s_PrintParameters.m_VertZoomOut = s_PrintrConfig.m_VertZoomOut;// = 1;//默认纵向放大倍数

s_PrintParameters.m_Liftpos = s_PrintrConfig.m_Liftpos;//  = 0;//默认打印机左限

s_PrintParameters.m_Righttpos = s_PrintrConfig.m_Righttpos;// = 384;//默认打印机右限

//s_PrintrConfig.m_BaudRate = 0;//串口机型波特率

g_ParTypeValue = PRINT_HEAD_LENGTH/s_PrintrConfig.m_SoftwarePart;//s_PrintrConfig.m_SoftwarePart = 4;//软件最大分区

g_THDeep = s_PrintrConfig.m_HotDeep;// = 0;//额外附加加热深度

g_PrinterDip = s_PrintrConfig.m_PrintSub;// = 3;//打印机细分度

s_PrintParameters.m_LineSpace = s_PrintrConfig.m_LineSpace;

s_PrintParameters.m_CharType = s_PrintrConfig.m_CharType;

}

/****************************************************************************

* 函数名：PrinterParameterInt()

* 功  能：打印机参数初始化

*

* 说  明：

****************************************************************************/

void  PrinterParameterInt(void)

{

int i;

s_PrintParameters.m_AlignUPDown = 0x8000;//=1表示下对齐

print_head_length = PRINT_HEAD_LENGTH;

s_PrintParameters.m_CharSpace = 0;//字间距默认为0

s_PrintParameters.m_CharType = TYPE_CHINESE;//默认为0表示打印的是汉字,小于80的表示打印的是字符

s_PrintParameters.m_ChineseCharType = TYPE_CHIN24X24;

s_PrintParameters.m_CharRepertoire = TYPE_ASC12X24;

s_PrintParameters.m_Charlist = 1;//如果打印ANK字符,默认打印字符集1

s_PrintParameters.m_HoriZoomOut = 1;//横向放大倍数为1

s_PrintParameters.m_VertZoomOut = 1;//纵向打印放大为1

s_PrintParameters.m_boldface = VALUE_DISABLE;//取消加粗模式

s_PrintParameters.m_Inverse = 0;//反白为0不反白

s_PrintParameters.m_Liftpos = 0;//左限的位置默认为起始位置0

s_PrintParameters.m_Righttpos = print_head_length;//打印机右限

s_PrintParameters.m_LineSpace = 3;//行间距默认为3

s_PrintParameters.m_PageLength = 255;//默认为255

s_PrintParameters.m_Pagecount = 0;//打印页长计数器为0

s_PrintParameters.m_positions = print_head_length;//打印机能打印的位置为打印头能够打印的点数

s_PrintParameters.m_PrintDir = PRINT_DIRECT_NOR;//正常台式打印机打印方向

s_PrintParameters.m_Prstate = 0;//打印机当前状态

s_PrintParameters.m_RotDegree = 0;//旋转角度默认为0度

s_PrintParameters.m_alignType = 0;//左对齐

for (i=0;i<20;i++)

{

s_PrintParameters.m_TabPos[i] = 0;//清空水平造表位置最小值为1

}

s_PrintParameters.m_TabCount = 0;//水平造表计数器为0

s_CodeBarCon.m_BarHigh = 50;//条码高度默认50

s_CodeBarCon.m_Barwide = 2;//宽度默认2

s_CodeBarCon.m_CharSpace = 2;//条码字符在上面

s_CodeBarCon.m_BarSpace = 0;

g_DriverAFlag = 0;

i = GetCurrentVol();

s_PrintParameters.m_CurMotoRunTime = VolRUNTime[i] - 100;//得到当前电压电机运行时间

s_PrintParameters.m_PrHeadHottime = VolTHSTB[i] - 200;//得到加热值

s_PrintParameters.m_CurMotoPWM = VolRUNPWM[i] - 100;//得到当前电压电机PWM值

s_PrintParameters.m_CurFreeTime = VolFreeTime[i];//得到当前电压空走纸步进时间

s_PrintParameters.m_VolTemAdd = VolTemAdd[i];

s_PrintParameters.m_VolTemSubb = VolTemSubb[i];

g_THDeep = 0;//默认增加加热深度0

g_ThType = 0;//加热类型

g_SpeedChoice = 0;//取消设定速度

g_CodePage = 255;//0;

g_InternationChar = 0;

s_PrintParameters.m_QRZoom = 0x02;

//得到配置表

ConfigToPrinter();

g_0D0AHigh = 24;//

g_HTCharSpace = 12;//根据打印的字符确定空格宽度

if(s_PrintrConfig.m_ChineseFont == PRINTTYPE_16)

{

g_0D0AHigh = 16;//

}

g_unicodeType = 0;//asc编码

}

#if 1

#define	CR_OPTWRE_Set			 ((unsigned int)0x00000200)

FLASH_Status ReadOutProtection(FunctionalState NewState)

{

FLASH_Status status = FLASH_COMPLETE;

/* Check the parameters */

assert_param(IS_FUNCTIONAL_STATE(NewState));

status = FLASH_WaitForLastOperation(EraseTimeout);

if(status == FLASH_COMPLETE)

{

/* Authorizes the small information block programming */

FLASH->OPTKEYR = FLASH_KEY1;

FLASH->OPTKEYR = FLASH_KEY2;

while((FLASH->CR&CR_OPTWRE_Set)!=CR_OPTWRE_Set )

{}

FLASH->CR |= CR_OPTER_Set;

FLASH->CR |= CR_STRT_Set;

/* Wait for last operation to be completed */

status = FLASH_WaitForLastOperation(EraseTimeout);

if(status == FLASH_COMPLETE)

{

/* if the erase operation is completed, disable the OPTER Bit */

FLASH->CR &= CR_OPTER_Reset;

/* Enable the Option Bytes Programming operation */

FLASH->CR |= CR_OPTPG_Set;

if(NewState != DISABLE)

{

OB->RDP = 0x00;

}

else

{

OB->RDP = RDP_Key;

}

/* Wait for last operation to be completed */

status = FLASH_WaitForLastOperation(EraseTimeout);

if(status != FLASH_TIMEOUT)

{

/* if the program operation is completed, disable the OPTPG Bit */

FLASH->CR &= CR_OPTPG_Reset;

}

}

else

{

if(status != FLASH_TIMEOUT)

{

/* Disable the OPTER Bit */

FLASH->CR &= CR_OPTER_Reset;

}

}

}

/* Return the protection operation Status */

return status;

}

#endif

/****************************************************************************

* 函数名：main()

* 功  能：主程序

*

* 说  明：

****************************************************************************/

int main()

{

unsigned int i;

TIM_TypeDef* TIM_Run;

//***********系统初始化**********************

g_ConfigRCC = 8;

FLASH_Unlock();

ReadOutProtection(ENABLE);

#ifdef RD_TYPE_SERIAL

mySPI_Init();//

MX_Resume();

//************************************************

p_PrCon = &s_PrintrConfig.m_InterFace;

//得到整个配置结构的所有数据

AT91F_DataFlashADDRRead(DL_PRINTER_PAR_ADDR+4,CONFIG_PAR_LENGTH,p_PrCon);//得到数据

g_ConfigRCC = s_PrintrConfig.m_BaudRate & 0xF;

#endif

for(i=0;i<0xfff;i++);

//初始化系统主时钟

RCC_Configuration();

//初始化中断

//初始化看门狗

WDG_Init();

NVIC_SetVectorTable(NVIC_VectTab_FLASH, 0x0); //设置中断向量在FLASH中

for(i=0;i<0xfff;i++);

#ifdef RD_TYPE_SERIAL

s_PrintParameters.m_InterFace = INTERFACE_SERIAL;

#endif

#ifdef RD_TYPE_PARALLEL

s_PrintParameters.m_InterFace = INTERFACE_PARALLEL;

#endif

//初始化SPI

mySPI_Init();//

MX_Resume();

//************************************************

Buffer_writer_I = &(s_ListParmeter_B.m_ListBuffer[700]);//设置指针位置

AT91F_DataFlashADDRRead(DL_PRINTER_PAR_ADDR,2,Buffer_Dataflash);//得到数据

g_ProFuncValue = CompileTime[0] << 8;

g_ProFuncValue |= CompileTime[1];

g_CurveCountFlag = CompileTime[3] << 8;

g_CurveCountFlag |= CompileTime[4];

if((Buffer_Dataflash[0] != g_ProFuncValue) || (Buffer_Dataflash[1] != g_CurveCountFlag))

{

//无参数配置需要初始化配置

PrinterConfigInt();

SavePrinterConfig();

}

p_PrCon = &s_PrintrConfig.m_InterFace;

//得到整个配置结构的所有数据

AT91F_DataFlashADDRRead(DL_PRINTER_PAR_ADDR+4,CONFIG_PAR_LENGTH,p_PrCon);//得到数据

//*****************系统配置*******************************

g_ProFuncValue = 0;

g_CurveCountFlag = 0;

//初始化IO

SystemGPIOInit();

//初始化PWM

PWM_Configuration();

g_receiveFlage = 0;

//初始化定时器

Timer_Configuration();

#ifdef RD_TYPE_SERIAL

FlowFlag = s_PrintrConfig.m_BaudRate;

#ifdef  SWITCH_BAUD

ReadSwStatus();

#endif

//初始化串口

#ifdef RD_USART1_EN

USART1_InitConfig(0);

#endif

#ifdef RD_USART2_EN

USART2_InitConfig(0);

#endif

#ifdef RD_USART3_EN

USART3_InitConfig(0);

#endif

#endif

//***************************************

#ifdef AUTO_FEED

g_printChar = 0;

#endif

//***************************************

#ifdef RD_TYPE_PARALLEL

//初始化并口

SET_H_BUSY_P;//置高忙信号,以防外部接口连接到客户设备后,外部设备发送数据误触发IRQ信号

SET_H_ACK;SET_L_ACK;SET_L_ACK;SET_L_ACK;SET_H_ACK;

SET_H_ACK;SET_L_ACK;SET_L_ACK;SET_L_ACK;SET_H_ACK;

SET_H_ACK;SET_L_ACK;SET_L_ACK;SET_L_ACK;SET_H_ACK;

Exit_Configuration();

SET_L_PE_P;//设置当前打印机有纸

#endif

IWDG_ReloadCounter(); //reload the watchdog 重装载看门狗定时器

#ifdef RD_MP3PRINTER

//E3系列的控制板使用TIM1作为系统定时器

TIM_Run = TIM1;

#endif

#ifdef RD_MICROPRINTER

//MP系列的控制板使用TIM1作为系统定时器

TIM_Run = TIM1;

#endif

#ifdef RD_OEMPRINTER

//OEM系列的控制板使用TIM2作为系统定时器

TIM_Run = TIM2;

#endif

#ifdef RD_EXPRINTER

//EF系列的控制板使用TIM2作为系统定时器

TIM_Run = TIM2;

#endif

//初始化ADC

ADC1_Configuration();

//初始化DMA

DMA_Configuration();

IWDG_ReloadCounter();  // reload the watchdog 重装载看门狗定时器

WRSR();

g_PrinterDip = 3;//默认4步一个点

//	g_PrinterDip = 1;//默认2步一个点

g_cruvefalg = 0xAAAA;

for(i=0;i<20;i++)

{

g_cruve_arry[i] = 0;

}

Erase_Sector = 0x300000; // 设置图形下载扇区比对初始值

#ifdef RD_OEMPRINTER

SET_L_POWER;//关闭驱动电源

SET_L_VCC_CON;//输出逻辑控制电源

#endif

//******************************************

g_Receive_count = 0;//清接收缓存计数器

Receive_count = 0;//清接收计数器

g_Order_count = 0;//清接取数寄存器

g_SaveDataToFlashAddr = ADDRPRO_OR;

g_GetDataFromFlashAddr = ADDRPRO_OR;

g_SaveDataOffset = 0;

g_GetDataOffset = 0;

g_InterFaceBufferCount = 0;

Order_Erase_Sector = 0x300000;// 设置打印命令存储扇区比对初始值

g_DumpReceiveFlage = 0;

//	g_DumpReceiveFlage = 0xAAAA;

g_BufferSwitch = 0;//初始化打印缓冲区切换标志

g_0D0AFlag =0;//清0d 0a标志

g_dotSwitch = 0;//预处理

s_PrintOrderstr.m_DataFlag = 0;//清空手动打印标志

s_PrintOrderstr.m_GetdataPos = 0;//清空手动打印计数器

//=====打印参数初始化

PrinterParameterInt();

//缓冲区的配置只在上电配置一次

s_PrintParameters.m_ReceiveBuffrWor =  s_PrintrConfig.m_ReceiveBuffrWor;//设置打印缓冲区标志

s_PrinterMOTOPar.m_PrintEnd = ENABLE_VAR;//打印机停机

s_PrinterMOTOPar.m_MutualFlag = DISABLE_VAR;//取消切换标志

s_PrinterMOTOPar.m_StopPrint = ENABLE_VAR;//停止打印禁止

#ifdef PRINTER_AND_USB

USB_Interrupts_Config();

IWDG_ReloadCounter();

Set_USBClock();

USB_Init();

#endif

g_PE_Standard = 0x50;//设置无纸时AD采样最低值

//设置打印机空闲

PrinterIdle();

SET_L_LED;

IWDG_ReloadCounter(); //reload the watchdog 重装载看门狗定时器

//======设置打印机缓冲区=================================

//	s_PrintParameters.m_ReceiveBuffrWor = RECEIVE_RAM;//RECEIVE_FLASH;

if(s_PrintParameters.m_ReceiveBuffrWor == RECEIVE_FLASH)

{

PrinterCore = PrinterCore_Flash;//得到取命令函数指针

p_OcAndGDAdr = &g_GetDataFromFlashAddr ;//g_Order_count

p_REAndSAcount = &g_InterFaceBufferCount; //Receive_count

p_SaveDrREcount = &g_SaveDataOffset;//保存数据的偏移量 g_Receive_count

}

else

{

PrinterCore = PrinterCore_RAM;

p_OcAndGDAdr = &g_Order_count ;//数据缓冲区buffer 位置偏移量 g_GetDataFromFlashAddr

p_REAndSAcount = &Receive_count; //接收数据寄存器g_InterFaceBufferCount

p_SaveDrREcount = &g_Receive_count;//保存数据的偏移量 g_SaveDataOffset

}

//======表格打印预处理=================================

for(i=0;i<BUFFER_SHORT_LENGTH;i++)

{

g_ListTable[i] = 0;

}

for(i=0;i<1000;i++);

g_ListSaveToFlashCount = 0;

g_ListPrintFinish = 0;

g_ListControlFor = 0;

g_PrintListOrNorm = 0;//不打印表格

//=======================================

g_SelfRunFlag = 0;//自检运行标志清零

g_PE_DN_Count = 0;//计数器清零

g_PE_UP_Count = 0;//计数器清零

g_PEDataS = (AD_Value[1] >> 4);

if(g_PEDataS > s_PrintrConfig.m_PE_LabValue_H)

{

g_PE_Staue = 1;

}

else

{

g_PE_Staue = 0;

}

Trg = 0;

Cont = 0;

key_Cont = 0;

g_PaperRunToOut = s_PrintrConfig.m_LabelLength + (s_PrintrConfig.m_BarPaGap /2 );

//=======================================

if(GET_KEY_LF == 0)

{

//打印自检使用FLASH存储,dunmp模式存储数据量比较大

s_PrintParameters.m_ReceiveBuffrWor = RECEIVE_FLASH;

PrinterCore = PrinterCore_Flash;//得到取命令函数指针

p_OcAndGDAdr = &g_GetDataFromFlashAddr ;//g_Order_count

p_REAndSAcount = &g_InterFaceBufferCount; //Receive_count

p_SaveDrREcount = &g_SaveDataOffset;//保存数据的偏移量 g_Receive_count

SysTick_Init();

p_LabData = &receive_bufer[1024];

for(i=0;i<0x200;i++)

{

p_LabData[i] = 0xFF;

}

g_SelfRunFlag = 0xAA;

SetSelftestData();

}

//=======================================

while (1)

{

ProcessStart:

__SETPRIMASK();

g_PrintStatusReg = PRASE_PROCESS;

__RESETPRIMASK();

if(g_BufferSwitch != 0)

p_ListPrease = &s_ListParmeter_B;

else

p_ListPrease = &s_ListParmeter_A;

HTCount = 0;

p_ListPrease->m_PrintLineSpace = s_PrintParameters.m_LineSpace;

p_ListPrease->m_HotTime = s_PrintParameters.m_PrHeadHottime;//得到打印头的加热时间

p_ListPrease->m_PrintDir = s_PrintParameters.m_PrintDir;//得到打印的方向

p_ListPrease->m_PrintLineHigh = 8;//默认一行的高度为8;

p_ListPrease->m_PrinterDip	= g_PrinterDip;

p_ListPrease->m_ThType = g_ThType;

p_ListPrease->m_MotoRunTime = g_SpeedChoice;//设置打印机运行速度

p_ListDataDot = &(p_ListPrease->m_ListBuffer[0]);//得到数据链表的首地址

p_IndexListData = &(p_ListPrease->m_ListIndex[0]);//得到链表索引的首地址

if (p_ListPrease->m_PrintDir != PRINT_DIRECT_NOR)

{

*p_IndexListData++= LIST_END;//如果为面板方向链表起始位为整个链表的末尾

}

p_ListPrease->m_MotoRunDir = MOTO_CLOCKWISE;//设置打印机正转

p_ListPrease->m_LableFeedFlag = 0;

p_ListPrease->m_LeftFreeDot = s_PrintParameters.m_Liftpos;

g_CalcPrintPos = s_PrintParameters.m_Liftpos;//打印数据的起始位置

g_SavePrintPos = g_CalcPrintPos;

m_SaverightPos = s_PrintParameters.m_Righttpos;//得到右限

g_alignType = s_PrintParameters.m_alignType;//得到对齐方式

for(i=0;i<BUFFER_SHORT_LENGTH;i++)

{

p_ListPrease->m_LineProArry[i] = g_ListTable[i];

}

ProcessLoop:

if (p_ListPrease->m_PrintDir != PRINT_DIRECT_NOR)

{

if (s_PrintParameters.m_AlignUPDown == 0)

{

m_UPDown = 0x8000;

}

else

{

m_UPDown = 0;

}

}

else

{

m_UPDown = s_PrintParameters.m_AlignUPDown;

}

//m_OrderSave = g_Order_count;

m_OrderSave = (*p_OcAndGDAdr);

//增加 unicode码的处理

ProUnicodeChar();

m_GetData = PrinterCore();//得到打印命令

if(g_DumpReceiveFlage == 0xAAAA)

{

if(m_GetData == 0x20)

{

if(g_SavePrintPos == g_CalcPrintPos)

{

m_GetData = 0;

}

}

}

Process_In_Loop:

m_ProcessFlag = PROCESS_CONTINUE;//默认为继续取命令

if(m_GetData > 0x1F)

{

m_ProcessStatus = __PrcessCharCh();

}

else

{

switch(m_GetData)

{

case 0x03:{ m_ProcessStatus = __03_Order(); }break; //协议

case 0x09:{ m_ProcessStatus = __HT_Order(); }break; //水平造表

case 0x0A:{ m_ProcessStatus = __OAOD_Order(); }break; //回车换行

case 0x0D:{ m_ProcessStatus = __OAOD_Order(); }break; //回车换行

case 0x0E:{ m_ProcessStatus = __SO_Order(); }break; //双倍宽

case 0x0C:{ m_ProcessStatus = __FF_Order(); }break; //黑标

case 0x12:{ m_ProcessStatus = __DC2_Order(); }break; //恢复倍宽

case 0x14:{ m_ProcessStatus = __DC4_Order(); }break; //恢复倍宽

case 0x18:{ m_ProcessStatus = __18_Order(); }break; //删除一行

case 0x1B:{ m_ProcessStatus = __ESC_Order(); }break; //解析ESC指令

case 0x1C:{ m_ProcessStatus = __FS_Order(); }break; //解析FS指令

case 0x1D:{ m_ProcessStatus = __GS_Order(); }break; //解析GS指令

default:

m_ProcessStatus = __OrderReceive();

}

}

if(m_ProcessStatus == PROCESS_START)

{

goto ProcessStart;

}

else if(m_ProcessStatus == PROCESS_IN_LOOP)

{

goto Process_In_Loop;

}

else if(m_ProcessStatus == PROCESS_LOOP)

{

goto ProcessLoop;

}

else if (m_ProcessStatus == PROCESS_CONTINUE)

{

goto ProcessLoop;

}

else if (m_ProcessStatus == PROCESS_FINISH)

{

//=========计算需要空出的空点=================================

if((g_THDeep & 0x8000) == 0)

{

p_ListPrease->m_HotTime = s_PrintParameters.m_PrHeadHottime + g_THDeep;//得到打印头的加热时间

}

else

{

p_ListPrease->m_HotTime = s_PrintParameters.m_PrHeadHottime - (g_THDeep & 0x7FFF);//得到打印头的加热时间

}