/****************************************************************************

* 文件名：Timer1PrintDataSend

* 功  能：向打印机头发送数据

* 入口参数：

* 说  明：在进入timer1中断后发送下一行需要打印的数据，PrintLineCount 打印行数计数器

*为了能不在向打印头串行发送数据的时候耽误CPU的时间，使用SSC这个单元发送数据

****************************************************************************/

void SendDICLK(unsigned short u16_data)

{

SaveToBuffer[SendBuffer_counter] = (u16_data | *(p_ListParmeters->m_LineProArry + SendBuffer_counter));

SendBuffer_counter++;

g_lineBitCount += CalintCount((u16_data | *(p_ListParmeters->m_LineProArry + SendBuffer_counter)));

}

void SendParData(unsigned short u16_anddata)

{

int i;

SendToSPI_counter = 0;

for(i=0;i<BUFFER_SHORT_LENGTH;i++)

{

while((SPI1->SR & SPI_FLAG_TXE) == 0);

SPI1->DR = (SendBufferToSPI[SendToSPI_counter++]& u16_anddata);

}

while((SPI1->SR & SPI_FLAG_TXE) == 0);

//	for(i=0;i<32;i++);

while((SPI1->SR & SPI_I2S_FLAG_BSY) != 0);

}

/****************************************************************************

* 文件名：Timer1PrintDataSend

* 功  能：向打印机头发送数据

* 入口参数：

* 说  明：在进入timer1中断后发送下一行需要打印的数据，PrintLineCount 打印行数计数器

*为了能不在向打印头串行发送数据的时候耽误CPU的时间，使用SSC这个单元发送数据

****************************************************************************/

void TIM3_IRQHandler(void)

{

p_PrintListPar			p_PrParmeters	;//链表结构指针

DataCon					m_SSCdata;//SSC数据发送结构

int 					dummy;						// temporary

unsigned int			m_Dotdata;

unsigned int			m_Tempdata;

//	int						m_xount;

unsigned short			addpar;

unsigned short			m_IndexCounter;//索引表

unsigned short			*p_PrinterData;//数据指针

unsigned short			m_GOrder;//命令

unsigned short			m_CharWide;//字符的宽度

unsigned short			m_WideZoom;//宽度放大

unsigned short			m_CharHigh;//字符的高度

unsigned short			m_HighZoom;//高度放大

unsigned short			m_calZe;//计算器

unsigned short			m_calR0;//计算器

unsigned short			m_alignSpace;//上下对齐和字间距

if ((TIM3->SR & TIM_IT_Update) == 0)

{

return;

}

//---------------清除中断标志----------------------------------------

//-------------禁止中断------------------------------------------

TIM_DISABLE(TIM3);

SendBuffer_counter = 0;

g_lineBitCount = 0;

g_SendFlag = DATA_IN_SEND;

SendBuffer_counter = 0;

m_SSCdata.m_data = 0;

m_SSCdata.m_len = 0;

if(s_PrinterMOTOPar.m_PrintLSbit == ENABLE_VAR)

{

for (dummy = BUFFER_SHORT_LENGTH;dummy > 0;dummy--)

{

SendDICLK(0);

}

g_SendFlag = DATA_SEND_OK;

return;

}

if(g_BufferSwitch != 0)

p_PrParmeters = &s_ListParmeter_A;

else

p_PrParmeters = &s_ListParmeter_B;

//---------------------先打印开头的空白点------------------

//==========================第一步========================================

if(p_PrParmeters->m_PrintDir == PRINT_DIRECT_NOR)

{

m_SSCdata.m_len = p_PrParmeters->m_LeftFreeDot + g_dotSwitch;

addpar = 1;

}

else

{

m_SSCdata.m_len = p_PrParmeters->m_RightFreeDot + g_dotSwitch;

addpar = 0xffff;

}

while(m_SSCdata.m_len >= 32)

{

SendDICLK(0);

SendDICLK(0);

m_SSCdata.m_len -= 32;

}

//==================================================================

m_IndexCounter = p_PrParmeters->m_PrintStartPos;

m_GOrder = p_PrParmeters->m_ListIndex[m_IndexCounter];

while (m_GOrder != LIST_END)

{

//===========开始打印数据

p_PrinterData = p_PrParmeters->m_ListBuffer + m_GOrder;//得到数据起始地址

m_GOrder = *p_PrinterData++;

if (m_GOrder == PRINT_DATA )

{

m_CharWide = ((*p_PrinterData) >> 4) & 0xff;

m_WideZoom = (*p_PrinterData++) & 0xf;

m_CharHigh = ((*p_PrinterData) >> 4) & 0xff;

m_HighZoom = (*p_PrinterData++) & 0xf;

m_alignSpace = *p_PrinterData++;

m_calZe = m_CharHigh * m_HighZoom;

//---------------判断可打印行数-------------------------

if ((m_alignSpace & 0x8000) != 0)

{

//-----------下标---------------------

dummy = g_PrintLineCount + m_calZe - p_PrParmeters->m_PrintLineHigh;//打印的行高

}

else

{

//-----------上标---------------------

if(g_PrintLineCount < m_calZe)

dummy = g_PrintLineCount;

else

dummy = -1;

}

m_alignSpace &= 0x7fff;//去掉最高位的上下对齐标志

//---------------打印空白点

m_calZe = m_CharWide * m_WideZoom;

if (dummy < 0)

{

if ((m_SSCdata.m_len + m_calZe) >= 32)

{

m_SSCdata.m_data = m_SSCdata.m_data << (32 - m_SSCdata.m_len);

m_SSCdata.m_len = m_calZe + m_SSCdata.m_len - 32;

SendDICLK(m_SSCdata.m_data >> 16);

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = 0;

while(m_SSCdata.m_len >= 32)

{

SendDICLK(0);

SendDICLK(0);

m_SSCdata.m_len -= 32;

}

}

else

{

m_SSCdata.m_len += m_calZe;

m_SSCdata.m_data = m_SSCdata.m_data << m_calZe;

}

if ((m_SSCdata.m_len + m_alignSpace) >= 32)

{

m_SSCdata.m_data = m_SSCdata.m_data << (32 - m_SSCdata.m_len);

m_SSCdata.m_len = m_alignSpace + m_SSCdata.m_len - 32;

SendDICLK(m_SSCdata.m_data >> 16);

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = 0;

while(m_SSCdata.m_len >= 32)

{

SendDICLK(0);

SendDICLK(0);

m_SSCdata.m_len -= 32;

}

}

else

{

m_SSCdata.m_len += m_alignSpace;

m_SSCdata.m_data = m_SSCdata.m_data << m_alignSpace;

}

m_IndexCounter += addpar;

m_GOrder = p_PrParmeters->m_ListIndex[m_IndexCounter];

continue;

}

//=--------------打印实际数据------------------------------------------

m_calR0 = (dummy * DiV_config[m_HighZoom]) >> 16;//得到行数

if (m_CharWide > 16)//15)

{

p_PrinterData  +=  (m_calR0 << 1);//得到点阵数据存放的起始位置

m_Dotdata = *p_PrinterData++;

m_Dotdata = (m_Dotdata << 16) | (*p_PrinterData++);

}

else

{

p_PrinterData  +=  m_calR0;

m_Dotdata = *p_PrinterData++;

}

if (m_WideZoom == 1)

{

if((m_SSCdata.m_len + m_calZe) <= 32)

{

m_SSCdata.m_len =  m_calZe + m_SSCdata.m_len;

m_SSCdata.m_data = (m_SSCdata.m_data << m_calZe) | m_Dotdata;

if (m_SSCdata.m_len == 32)

{

SendDICLK(m_SSCdata.m_data >> 16);

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = 0;

m_SSCdata.m_len = 0;

}

if (m_SSCdata.m_len == 16)

{

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = 0;

m_SSCdata.m_len = 0;

}

}

else

{

m_SSCdata.m_data = m_SSCdata.m_data << (32 - m_SSCdata.m_len);

m_calR0 = m_calZe + m_SSCdata.m_len -32;

m_Tempdata = m_Dotdata >> m_calR0;

m_SSCdata.m_data |= m_Tempdata;

SendDICLK(m_SSCdata.m_data >> 16);

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = m_Dotdata;

m_SSCdata.m_len = m_calR0;

}

if((m_SSCdata.m_len + m_alignSpace) <= 32)

{

m_SSCdata.m_len =  m_alignSpace + m_SSCdata.m_len;

m_SSCdata.m_data = (m_SSCdata.m_data << m_alignSpace) ;

if (m_SSCdata.m_len == 32)

{

SendDICLK(m_SSCdata.m_data >> 16);

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = 0;

m_SSCdata.m_len = 0;

}

if (m_SSCdata.m_len == 16)

{

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = 0;

m_SSCdata.m_len = 0;

}

}

else

{

m_SSCdata.m_data = m_SSCdata.m_data << (32 - m_SSCdata.m_len);

m_calR0 = m_alignSpace + m_SSCdata.m_len -32;

SendDICLK(m_SSCdata.m_data >> 16);

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = 0;

m_SSCdata.m_len = 0;

while(m_calR0 >= 32)

{

SendDICLK(0);

SendDICLK(0);

m_calR0 -= 32;

}

m_SSCdata.m_len = m_calR0;

}

}

else

{

dummy = m_CharWide;

while (dummy > 4)

{

m_Tempdata = (m_Dotdata >> (dummy - 4)) & 0xf;

m_calR0 = m_WideZoom << 2;

m_Tempdata = m_Tempdata + ((m_WideZoom -1)<< 4);

m_Tempdata = Config_zoom[m_Tempdata];//得到放大后的数据

if((m_SSCdata.m_len + m_calR0) <= 32)

{

m_SSCdata.m_len =  m_calR0 + m_SSCdata.m_len;

m_SSCdata.m_data = (m_SSCdata.m_data << m_calR0) | m_Tempdata;

if (m_SSCdata.m_len == 32)

{

SendDICLK(m_SSCdata.m_data >> 16);

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = 0;

m_SSCdata.m_len = 0;

}

if (m_SSCdata.m_len == 16)

{

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = 0;

m_SSCdata.m_len = 0;

}

}

else

{

m_SSCdata.m_data = m_SSCdata.m_data << (32 - m_SSCdata.m_len);

m_calR0 = m_calR0 + m_SSCdata.m_len -32;

m_SSCdata.m_data |= m_Tempdata >> m_calR0;

SendDICLK(m_SSCdata.m_data >> 16);

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = m_Tempdata;

m_SSCdata.m_len = m_calR0;

}

dummy -= 4;

}

if (dummy != 0)

{

m_Tempdata = m_Dotdata  & 0xf;

m_calR0 = m_WideZoom * dummy;//得到移位大小

m_Tempdata = m_Tempdata + ((m_WideZoom -1)<< 4);

m_Tempdata = Config_zoom[m_Tempdata];//得到放大后的数据

if((m_SSCdata.m_len + m_calR0) <= 32)

{

m_SSCdata.m_len =  m_calR0 + m_SSCdata.m_len;

m_SSCdata.m_data = (m_SSCdata.m_data << m_calR0) | m_Tempdata;

if (m_SSCdata.m_len == 32)

{

SendDICLK(m_SSCdata.m_data >> 16);

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = 0;

m_SSCdata.m_len = 0;

}

if (m_SSCdata.m_len == 16)

{

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = 0;

m_SSCdata.m_len = 0;

}

}

else

{

m_SSCdata.m_data = m_SSCdata.m_data << (32 - m_SSCdata.m_len);

m_calR0 = m_calR0 + m_SSCdata.m_len -32;

m_SSCdata.m_data |= m_Tempdata >> m_calR0;

SendDICLK(m_SSCdata.m_data >> 16);

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = m_Tempdata;

m_SSCdata.m_len = m_calR0;

}

}

if((m_SSCdata.m_len + m_alignSpace) <= 32)

{

m_SSCdata.m_len =  m_alignSpace + m_SSCdata.m_len;

m_SSCdata.m_data = (m_SSCdata.m_data << m_alignSpace) ;

if (m_SSCdata.m_len == 32)

{

SendDICLK(m_SSCdata.m_data >> 16);

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = 0;

m_SSCdata.m_len = 0;

}

if (m_SSCdata.m_len == 16)

{

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = 0;

m_SSCdata.m_len = 0;

}

}

else

{

m_SSCdata.m_data = m_SSCdata.m_data << (32 - m_SSCdata.m_len);

m_calR0 = m_alignSpace + m_SSCdata.m_len -32;

SendDICLK(m_SSCdata.m_data >> 16);

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = 0;

m_SSCdata.m_len = 0;

while(m_calR0 >= 32)

{

SendDICLK(0);

SendDICLK(0);

m_calR0 -= 32;

}

m_SSCdata.m_len = m_calR0;

}

}

}

else

{

//处理 空点 和图形 曲线

//========================================

if (m_GOrder == PRINT_CURVE )

{

p_PrinterData++; //得到打印的方向

dummy = BUFFER_SHORT_LENGTH;//print_head_length >> 4;

for (;dummy > 0;dummy--)

{

SendDICLK(*p_PrinterData++ );

}

m_SSCdata.m_len =0;

m_calR0 = 0;

}

if (m_GOrder == PRINT_SPACE )

{

m_CharWide = *p_PrinterData;//得到打印空点的宽度

if((m_SSCdata.m_len + m_CharWide) <= 32)

{

m_SSCdata.m_len =  m_CharWide + m_SSCdata.m_len;

m_SSCdata.m_data = (m_SSCdata.m_data << m_CharWide) ;

if (m_SSCdata.m_len == 32)

{

SendDICLK(m_SSCdata.m_data >> 16);

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = 0;

m_SSCdata.m_len = 0;

}

if (m_SSCdata.m_len == 16)

{

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = 0;

m_SSCdata.m_len = 0;

}

}

else

{

m_SSCdata.m_data = m_SSCdata.m_data << (32 - m_SSCdata.m_len);

m_calR0 = m_CharWide + m_SSCdata.m_len -32;

SendDICLK(m_SSCdata.m_data >> 16);

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = 0;

m_SSCdata.m_len = 0;

while(m_calR0 >= 32)

{

SendDICLK(0);

SendDICLK(0 );

m_calR0 -= 32;

}

m_SSCdata.m_len = m_calR0;

}

}

if (m_GOrder == PRINT_MAP )

{

m_CharWide = ((*p_PrinterData) >> 4) & 0x3ff;

m_WideZoom = (*p_PrinterData++) & 0xf;

m_CharHigh = ((*p_PrinterData) >> 4) & 0x3ff;

m_HighZoom = (*p_PrinterData++) & 0xf;

m_alignSpace = *p_PrinterData++;

m_calZe = m_CharHigh * m_HighZoom;

//---------------判断可打印行数-------------------------

if ((m_alignSpace & 0x8000) != 0)

{

//-----------下标---------------------

dummy = g_PrintLineCount + m_calZe - p_PrParmeters->m_PrintLineHigh;//打印的行高

}

else

{

//-----------上标---------------------

if(g_PrintLineCount < m_calZe)

dummy = g_PrintLineCount;

else

dummy = -1;

}

m_alignSpace &= 0x7fff;//去掉最高位的上下对齐标志

//---------------打印空白点

m_calZe = m_CharWide * m_WideZoom;

if (dummy < 0)

{

if ((m_SSCdata.m_len + m_calZe) >= 32)

{

m_SSCdata.m_data = m_SSCdata.m_data << (32 - m_SSCdata.m_len);

m_SSCdata.m_len = m_calZe + m_SSCdata.m_len - 32;

SendDICLK(m_SSCdata.m_data >> 16);

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = 0;

while(m_SSCdata.m_len >= 32)

{

SendDICLK(0);

SendDICLK(0);

m_SSCdata.m_len -= 32;

}

}

else

{

m_SSCdata.m_len += m_calZe;

m_SSCdata.m_data = m_SSCdata.m_data << m_calZe;

}

if ((m_SSCdata.m_len + m_alignSpace) >= 32)

{

m_SSCdata.m_data = m_SSCdata.m_data << (32 - m_SSCdata.m_len);

m_SSCdata.m_len = m_alignSpace + m_SSCdata.m_len - 32;

SendDICLK(m_SSCdata.m_data >> 16);

SendDICLK(m_SSCdata.m_data );

while(m_SSCdata.m_len >= 32)

{

SendDICLK(0);

SendDICLK(0);

m_SSCdata.m_len -= 32;

}

}

else

{

m_SSCdata.m_len += m_alignSpace;

m_SSCdata.m_data = m_SSCdata.m_data << m_alignSpace;

}

m_IndexCounter += addpar;

m_GOrder = p_PrParmeters->m_ListIndex[m_IndexCounter];

continue;

}

//=--------------打印实际数据------------------------------------------

m_calR0 = (dummy * DiV_config[m_HighZoom]) >> 16;//得到行数

//---------得到图形宽度字节

m_calZe = m_CharWide >> 4;

if ((m_CharWide & 0xf) != 0)

{

m_calZe++;

}

//数据起始地址

//--------------------

p_PrinterData  +=  m_calR0 * m_calZe;

//--------------------

dummy = m_CharWide;

while (m_CharWide > 16)

{

m_Dotdata = *p_PrinterData++;

dummy = 16;

while(dummy!=0)

{

m_Tempdata = (m_Dotdata >> (dummy - 4)) & 0xf;

m_calR0 = m_WideZoom << 2;

m_Tempdata = m_Tempdata + ((m_WideZoom -1)<< 4);//

m_Tempdata = Config_zoom[m_Tempdata];//得到放大后的数据

//-------------发送数据

if((m_SSCdata.m_len + m_calR0) <= 32)

{

m_SSCdata.m_len =  m_calR0 + m_SSCdata.m_len;

m_SSCdata.m_data = (m_SSCdata.m_data << m_calR0) | m_Tempdata;

if (m_SSCdata.m_len == 32)

{

SendDICLK(m_SSCdata.m_data >> 16);

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = 0;

m_SSCdata.m_len = 0;

}

if (m_SSCdata.m_len == 16)

{

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = 0;

m_SSCdata.m_len = 0;

}

}

else

{

m_SSCdata.m_data = m_SSCdata.m_data << (32 - m_SSCdata.m_len);

m_calR0 = m_calR0 + m_SSCdata.m_len -32;

m_SSCdata.m_data |= m_Tempdata >> m_calR0;

SendDICLK(m_SSCdata.m_data >> 16);

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = m_Tempdata;

m_SSCdata.m_len = m_calR0;

}

dummy -= 4;

}

m_CharWide -= 16;

}

if (m_CharWide > 0)

{

//处理剩余数据

m_Dotdata = *p_PrinterData++;

dummy = m_CharWide;

while(dummy!=0)

{

if (dummy > 4)

{

m_Tempdata = (m_Dotdata >> (dummy - 4)) & 0xf;

m_calR0 = m_WideZoom << 2;

dummy -= 4;

}

else

{

m_Tempdata = m_Dotdata & 0xf;

m_calR0 = m_WideZoom * dummy;

dummy = 0;

}

m_Tempdata = m_Tempdata + ((m_WideZoom -1)<< 4);//

m_Tempdata = Config_zoom[m_Tempdata];//得到放大后的数据

//-------------发送数据

if((m_SSCdata.m_len + m_calR0) <= 32)

{

m_SSCdata.m_len =  m_calR0 + m_SSCdata.m_len;

m_SSCdata.m_data = (m_SSCdata.m_data << m_calR0) | m_Tempdata;

if (m_SSCdata.m_len == 32)

{

SendDICLK(m_SSCdata.m_data >> 16);

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = 0;

m_SSCdata.m_len = 0;

}

if (m_SSCdata.m_len == 16)

{

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = 0;

m_SSCdata.m_len = 0;

}

}

else

{

m_SSCdata.m_data = m_SSCdata.m_data << (32 - m_SSCdata.m_len);

m_calR0 = m_calR0 + m_SSCdata.m_len -32;

m_SSCdata.m_data |= m_Tempdata >> m_calR0;

SendDICLK(m_SSCdata.m_data >> 16);

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = m_Tempdata;

m_SSCdata.m_len = m_calR0;

}

}

}

if((m_SSCdata.m_len + m_alignSpace) <= 32)

{

m_SSCdata.m_len =  m_alignSpace + m_SSCdata.m_len;

m_SSCdata.m_data = (m_SSCdata.m_data << m_alignSpace) ;

if (m_SSCdata.m_len == 32)

{

SendDICLK(m_SSCdata.m_data >> 16);

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = 0;

m_SSCdata.m_len = 0;

}

if (m_SSCdata.m_len == 16)

{

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = 0;

m_SSCdata.m_len = 0;

}

}

else

{

m_SSCdata.m_data = m_SSCdata.m_data << (32 - m_SSCdata.m_len);

m_calR0 = m_alignSpace + m_SSCdata.m_len -32;

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = 0;

m_SSCdata.m_len = 0;

while(m_calR0 >= 32)

{

SendDICLK(0);

SendDICLK(0);

m_calR0 -= 32;

}

m_SSCdata.m_len = m_calR0;

}

}

}

m_IndexCounter += addpar;

m_GOrder = p_PrParmeters->m_ListIndex[m_IndexCounter];

}

//*********************结束******************************************

if(p_PrParmeters->m_PrintDir == PRINT_DIRECT_NOR)

{

m_calR0 = p_PrParmeters->m_RightFreeDot + g_dotSwitch ;

}

else

{

m_calR0 = p_PrParmeters->m_LeftFreeDot + g_dotSwitch;

}

if((m_SSCdata.m_len + m_calR0) == 0)

{

g_SendFlag = DATA_SEND_OK;

return;

}

if((m_SSCdata.m_len + m_calR0) <= 32)

{

m_SSCdata.m_len =  m_calR0 + m_SSCdata.m_len;

m_SSCdata.m_data = (m_SSCdata.m_data << m_calR0) ;

if (m_SSCdata.m_len == 32)

{

SendDICLK(m_SSCdata.m_data >> 16);

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = 0;

m_SSCdata.m_len = 0;

}

if (m_SSCdata.m_len == 16)

{

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = 0;

m_SSCdata.m_len = 0;

}

}

else

{

m_SSCdata.m_data = m_SSCdata.m_data << (32 - m_SSCdata.m_len);

m_calR0 = m_calR0 + m_SSCdata.m_len -32;

SendDICLK(m_SSCdata.m_data >> 16);

SendDICLK(m_SSCdata.m_data );

m_SSCdata.m_data = 0;

m_SSCdata.m_len = 0;

while(m_calR0 >= 16)

{

SendDICLK(0);

m_calR0 -= 16;

}

m_SSCdata.m_len = m_calR0;

}

//**************************************************************

g_SendFlag = DATA_SEND_OK;

}

void SetSelftestData(void)

{

unsigned short i,com,m_DaSpave;

unsigned int m_judgeyear;

s_PrintParameters.m_LineSpace = 6;

IWDG_ReloadCounter();

i =0;

while(CompileDate[i])

{

receive_bufer[(*p_REAndSAcount)++] = CompileDate[i++];

}

receive_bufer[(*p_REAndSAcount)++] = 0x20;

i =0;

while(CompileTime[i])

{

receive_bufer[(*p_REAndSAcount)++] = CompileTime[i++];

}

receive_bufer[(*p_REAndSAcount)++] = 0x0D;

i = 0;

m_DaSpave = (*p_REAndSAcount);//得到序列号起始位置

while(SelfTestSerial[i])

{

receive_bufer[(*p_REAndSAcount)++] = SelfTestSerial[i++];

}

m_judgeyear = 0;

m_judgeyear = CompileDate[0]<<16;

m_judgeyear |= CompileDate[1] << 8;

m_judgeyear |= CompileDate[2];

receive_bufer[m_DaSpave+9] = '2';

switch(m_judgeyear)

{

case 0x4A616E:

receive_bufer[m_DaSpave+10] = '1';

break;

case 0x466562:

receive_bufer[m_DaSpave+10] = '2';

break;

case 0x4D6172:

receive_bufer[m_DaSpave+10] = '3';

break;

case 0x417072:

receive_bufer[m_DaSpave+10] = '4';

break;

case 0x4D6179:

receive_bufer[m_DaSpave+10] = '5';

break;

case 0x4A756E:

receive_bufer[m_DaSpave+10] = '6';

break;

case 0x4A756C:

receive_bufer[m_DaSpave+10] = '7';

break;

case 0x417567:

receive_bufer[m_DaSpave+10] = '8';

break;

case 0x536570:

receive_bufer[m_DaSpave+10] = '9';

break;

case 0x4F6374:

receive_bufer[m_DaSpave+9] = '3';

receive_bufer[m_DaSpave+10] = '0';

break;

case 0x4E6F76:

receive_bufer[m_DaSpave+9] = '3';

receive_bufer[m_DaSpave+10] = '1';

break;

case 0x446563:

receive_bufer[m_DaSpave+9] = '3';

receive_bufer[m_DaSpave+10] = '2';

break;

}

receive_bufer[m_DaSpave+11] = CompileDate[4];

if(CompileDate[4] == ' ')

{

receive_bufer[m_DaSpave+11] = '0'	  ;

}

receive_bufer[m_DaSpave+12] = CompileDate[5];

receive_bufer[m_DaSpave+7] = CompileDate[9] + 5;

receive_bufer[m_DaSpave+8] = CompileDate[10];

receive_bufer[m_DaSpave+13] = CompileTime[0];

receive_bufer[m_DaSpave+14] = CompileTime[1];

receive_bufer[m_DaSpave+15] = CompileTime[3];

receive_bufer[m_DaSpave+16] = CompileTime[4];

if(s_PrintParameters.m_InterFace == INTERFACE_SERIAL)

{

i = 0;

while(SelfTestD_1[i])

{

receive_bufer[(*p_REAndSAcount)++] = SelfTestD_1[i++];

}

com = FlowFlag & 0x0F;

switch(com)

{

case 0x0:

//150

i =0;

while(SelfTestD_150[i])

{

receive_bufer[(*p_REAndSAcount)++] = SelfTestD_150[i++];

}

break;

case 0x1:

//300

i =0;

while(SelfTestD_300[i])

{

receive_bufer[(*p_REAndSAcount)++] = SelfTestD_300[i++];

}

break;

case 0x2:

//600

i =0;

while(SelfTestD_600[i])

{

receive_bufer[(*p_REAndSAcount)++] = SelfTestD_600[i++];

}

break;

case 0x3:

//1200

i =0;

while(SelfTestD_1200[i])

{

receive_bufer[(*p_REAndSAcount)++] = SelfTestD_1200[i++];

}

break;

case 0x4:

//2400

i =0;

while(SelfTestD_2400[i])

{

receive_bufer[(*p_REAndSAcount)++] = SelfTestD_2400[i++];

}

break;

case 0x5:

//4800

i =0;

while(SelfTestD_4800[i])

{

receive_bufer[(*p_REAndSAcount)++] = SelfTestD_4800[i++];

}

break;

case 0x6:

//9600

i =0;

while(SelfTestD_9600[i])

{

receive_bufer[(*p_REAndSAcount)++] = SelfTestD_9600[i++];

}

break;

case 0x7:

//19200  0000

i =0;

while(SelfTestD_19200[i])

{

receive_bufer[(*p_REAndSAcount)++] = SelfTestD_19200[i++];

}

break;

case 0x8:

//4800

i =0;

while(SelfTestD_38400[i])

{

receive_bufer[(*p_REAndSAcount)++] = SelfTestD_38400[i++];

}

break;

case 0x9:

//9600

i =0;

while(SelfTestD_57600[i])

{

receive_bufer[(*p_REAndSAcount)++] = SelfTestD_57600[i++];

}

break;

case 0xA:

//19200  0000

i =0;

while(SelfTestD_115200[i])

{

receive_bufer[(*p_REAndSAcount)++] = SelfTestD_115200[i++];

}

break;

}

switch(FlowFlag & 0x30)

{

case 0x00:

i =0;

while(SelfTestD_8N1[i])

{

receive_bufer[(*p_REAndSAcount)++] = SelfTestD_8N1[i++];

}

break;

case 0x20:

i =0;

while(SelfTestD_8O1[i])

{

receive_bufer[(*p_REAndSAcount)++] = SelfTestD_8O1[i++];

}

break;

case 0x10:

i =0;

while(SelfTestD_8E1[i])

{

receive_bufer[(*p_REAndSAcount)++] = SelfTestD_8E1[i++];

}

break;

case 0x30:

i =0;

while(SelfTestD_7E1[i])

{

receive_bufer[(*p_REAndSAcount)++] = SelfTestD_7E1[i++];

}

break;

}

if((FlowFlag & 0x40) == 0)

{

i =0;

while(SelfTestD_Ctrl[i])

{

receive_bufer[(*p_REAndSAcount)++] = SelfTestD_Ctrl[i++];

}

}

else

{

i =0;

while(SelfTestD_XFCtrl[i])

{

receive_bufer[(*p_REAndSAcount)++] = SelfTestD_XFCtrl[i++];

}

}

}

else

{

i = 0;

while(SelfTestD_P[i])

{

receive_bufer[(*p_REAndSAcount)++] = SelfTestD_P[i++];

}

}

if(s_PrintrConfig.m_ChineseFont == PRINTTYPE_24)

{

i =0;

while(SelfTestD_2[i])

{

receive_bufer[(*p_REAndSAcount)++] = SelfTestD_2[i++];

}

if(s_PrintrConfig.m_ChineseLib == TYPE_GB2312)

{

i =0;

while(SelfTestD_24_GB2312[i])

{

receive_bufer[(*p_REAndSAcount)++] = SelfTestD_24_GB2312[i++];

}

}

else

{

i =0;

while(SelfTestD_24_GBK[i])

{

receive_bufer[(*p_REAndSAcount)++] = SelfTestD_24_GBK[i++];

}

}

}

else

{

i =0;

while(SelfTestD_2_1[i])

{

receive_bufer[(*p_REAndSAcount)++] = SelfTestD_2_1[i++];

}

if(s_PrintrConfig.m_ChineseLib == TYPE_GB2312)

{

i =0;

while(SelfTestD_16_GB2312[i])

{

receive_bufer[(*p_REAndSAcount)++] = SelfTestD_16_GB2312[i++];

}

}

else

{

i =0;

while(SelfTestD_16_GBK[i])

{

receive_bufer[(*p_REAndSAcount)++] = SelfTestD_16_GBK[i++];

}

}

}

i =0;

while(SelfTest_S[i] != 0xff)

{

receive_bufer[(*p_REAndSAcount)++] = SelfTest_S[i++];

}

for (i=0x20;i<0x100;i++)

{

receive_bufer[(*p_REAndSAcount)++] = i;

}

receive_bufer[(*p_REAndSAcount)++] = 0x0D;

receive_bufer[(*p_REAndSAcount)++] = 0x0D;

i =0;

while(SelfTest_COM[i])

{

receive_bufer[(*p_REAndSAcount)++] = SelfTest_COM[i++];

}

receive_bufer[(*p_REAndSAcount)++] = 0x0D;

receive_bufer[(*p_REAndSAcount)++] = 0x0D;

receive_bufer[(*p_REAndSAcount)++] = 0x12;

receive_bufer[(*p_REAndSAcount)++] = 0xAD;//设置DUMP模式

receive_bufer[(*p_REAndSAcount)++] = 0x1B;

receive_bufer[(*p_REAndSAcount)++] = 0x40;

receive_bufer[(*p_REAndSAcount)++] = 0x1B;

receive_bufer[(*p_REAndSAcount)++] = 0x31;

receive_bufer[(*p_REAndSAcount)++] = 0x06;

receive_bufer[(*p_REAndSAcount)++] = 0x1B;//计算标签值

receive_bufer[(*p_REAndSAcount)++] = 0xBC;

receive_bufer[(*p_REAndSAcount)++] = 0x53;

receive_bufer[(*p_REAndSAcount)++] = 0xAA;

(*p_SaveDrREcount) = (*p_REAndSAcount);

IWDG_ReloadCounter();

}

void SetSerialContrl(unsigned char baudvalue)

{

const char*  p_SetDataParity;

unsigned char i;

switch(baudvalue&0xF)

{

case 0x0:

p_SetDataParity = SelfTestD_Ctrl;

break;

case 0x1:

p_SetDataParity = SelfTestD_XFCtrl;

break;

}

i =0;

while(p_SetDataParity[i])

{

(*p_REAndSAcount)++;//缓冲区得到的数据个数

receive_bufer[(*p_SaveDrREcount)++] =p_SetDataParity[i++];

(*p_SaveDrREcount) = (*p_SaveDrREcount) & MASK_DATA;

}

for(i=0;i<4;i++)

{

(*p_REAndSAcount)++;//缓冲区得到的数据个数

receive_bufer[(*p_SaveDrREcount)++] = 0x0D;

(*p_SaveDrREcount) = (*p_SaveDrREcount) & MASK_DATA;

}

}

void DefaultSet(void)

{

unsigned char i;

i =0;

while(Setresort[i])

{

(*p_REAndSAcount)++;//缓冲区得到的数据个数

receive_bufer[(*p_SaveDrREcount)++] =Setresort[i++];

(*p_SaveDrREcount) = (*p_SaveDrREcount) & MASK_DATA;

}

for(i=0;i<4;i++)

{

(*p_REAndSAcount)++;//缓冲区得到的数据个数

receive_bufer[(*p_SaveDrREcount)++] = 0x0D;

(*p_SaveDrREcount) = (*p_SaveDrREcount) & MASK_DATA;

}

}

void SetSoftwarePart(unsigned char baudvalue)

{

const char*  p_SetDataParity;

unsigned char i;

i =0;

p_SetDataParity = Setenergy;

while(p_SetDataParity[i])

{

(*p_REAndSAcount)++;//缓冲区得到的数据个数

receive_bufer[(*p_SaveDrREcount)++] =p_SetDataParity[i++];

(*p_SaveDrREcount) = (*p_SaveDrREcount) & MASK_DATA;

}

(*p_REAndSAcount)++;//缓冲区得到的数据个数

receive_bufer[(*p_SaveDrREcount)++] = (6 - baudvalue) + 0x30;

(*p_SaveDrREcount) = (*p_SaveDrREcount) & MASK_DATA;

for(i=0;i<4;i++)

{

(*p_REAndSAcount)++;//缓冲区得到的数据个数

receive_bufer[(*p_SaveDrREcount)++] = 0x0D;

(*p_SaveDrREcount) = (*p_SaveDrREcount) & MASK_DATA;

}

}

void SetPrintFont(unsigned char baudvalue)

{

const char*  p_SetDataParity;

unsigned char i;

switch(baudvalue&0xF)

{

case 0x0:

p_SetDataParity = GB2312_24x24;

break;

case 0x1:

p_SetDataParity = GB18030_24x24;

break;

case 0x2:

p_SetDataParity = GB2312_16x16;

break;

}

i =0;

while(p_SetDataParity[i])

{

(*p_REAndSAcount)++;//缓冲区得到的数据个数

receive_bufer[(*p_SaveDrREcount)++] =p_SetDataParity[i++];

(*p_SaveDrREcount) = (*p_SaveDrREcount) & MASK_DATA;

}

for(i=0;i<4;i++)

{

(*p_REAndSAcount)++;//缓冲区得到的数据个数

receive_bufer[(*p_SaveDrREcount)++] = 0x0D;

(*p_SaveDrREcount) = (*p_SaveDrREcount) & MASK_DATA;

}

}

void SetPrtintDir(unsigned char baudvalue)

{

const char*  p_SetDataParity;

unsigned char i;

switch(baudvalue&0xF)

{

case 0x0:

p_SetDataParity = Self_Forward;

break;

case 0x1:

p_SetDataParity = Self_Reverse;

break;

}

i =0;

while(p_SetDataParity[i])

{

(*p_REAndSAcount)++;//缓冲区得到的数据个数

receive_bufer[(*p_SaveDrREcount)++] =p_SetDataParity[i++];

(*p_SaveDrREcount) = (*p_SaveDrREcount) & MASK_DATA;

}

for(i=0;i<4;i++)

{

(*p_REAndSAcount)++;//缓冲区得到的数据个数

receive_bufer[(*p_SaveDrREcount)++] = 0x0D;

(*p_SaveDrREcount) = (*p_SaveDrREcount) & MASK_DATA;

}

}

void SetPrtintDataParity(unsigned char baudvalue)

{

const char*  p_SetDataParity;

unsigned char i;

SetDataBaud(g_cruve_arry[1]);

switch(baudvalue&0x3)

{

case 0x0:

p_SetDataParity = SelfTestD_8N1;

break;

case 0x1:

p_SetDataParity = SelfTestD_8O1;

break;

case 0x2:

p_SetDataParity = SelfTestD_8E1;

break;

case 0x3:

p_SetDataParity = SelfTestD_7E1;

break;

case 0x4:

p_SetDataParity = SelfTestD_8N1;

}

i =0;

while(p_SetDataParity[i])

{

(*p_REAndSAcount)++;//缓冲区得到的数据个数

receive_bufer[(*p_SaveDrREcount)++] =p_SetDataParity[i++];

(*p_SaveDrREcount) = (*p_SaveDrREcount) & MASK_DATA;

}

for(i=0;i<4;i++)

{

(*p_REAndSAcount)++;//缓冲区得到的数据个数

receive_bufer[(*p_SaveDrREcount)++] = 0x0D;

(*p_SaveDrREcount) = (*p_SaveDrREcount) & MASK_DATA;

}

}

void SetDataBaud(unsigned char baudvalue)

{

const char*  p_SetDataBaude;

unsigned char i;

switch(baudvalue&0xF)

{

case 0x0:

//150

p_SetDataBaude = SelfTestD_150;

break;

case 0x1:

//300

p_SetDataBaude = SelfTestD_300;

break;

case 0x2:

//600

p_SetDataBaude = SelfTestD_600;

break;

case 0x3:

//1200

p_SetDataBaude = SelfTestD_1200;

break;

case 0x4:

//2400

p_SetDataBaude = SelfTestD_2400;

break;

case 0x5:

//4800

p_SetDataBaude = SelfTestD_4800;

break;

case 0x6:

//9600

p_SetDataBaude = SelfTestD_9600;

break;

case 0x7:

//19200  0000

p_SetDataBaude = SelfTestD_19200;

break;

case 0x8:

//19200  0000

p_SetDataBaude = SelfTestD_38400;

break;

case 0x9:

//19200  0000

p_SetDataBaude = SelfTestD_57600;

break;

case 0xA:

//19200  0000

p_SetDataBaude = SelfTestD_115200;

break;

}

i =0;

while(p_SetDataBaude[i])

{

(*p_REAndSAcount)++;//缓冲区得到的数据个数

receive_bufer[(*p_SaveDrREcount)++] =p_SetDataBaude[i++];

(*p_SaveDrREcount) = (*p_SaveDrREcount) & MASK_DATA;

}

}

void SetPrtintDataBaud(unsigned char baudvalue)

{

unsigned char i;

SetDataBaud(baudvalue);

for(i=0;i<4;i++)

{

(*p_REAndSAcount)++;//缓冲区得到的数据个数

receive_bufer[(*p_SaveDrREcount)++] = 0x0D;

(*p_SaveDrREcount) = (*p_SaveDrREcount) & MASK_DATA;

}

}

/****************************************************************************

* 函数名： PrinterLPS()

* 功  能：检测缺纸

****************************************************************************/

void PrinterLPS(void)

{

unsigned int count,i;

g_LPSStatus = AD_Value[1]>>4;

IWDG_ReloadCounter(); //reload the watchdog

if(g_LPSStatus < g_PE_Standard)

{

for(count = 0;count<0xA50;count++);

g_LPSStatus = AD_Value[1]>>4;

if(g_LPSStatus > g_PE_Standard)

{

return;

}

for(count = 0;count<0x200;count++);

g_LPSStatus = AD_Value[1]>>4;

if(g_LPSStatus > g_PE_Standard)

{

return;

}

//	g_PaperEnd = 0xAA55;

lps_feed:

while(g_LPSStatus < g_PE_Standard)

{

#ifdef RD_TYPE_PARALLEL

#ifdef RD_PARALLEL_PE

SET_H_PE_P;

#endif

#endif

g_LPSStatus = AD_Value[1]>>4;

IWDG_ReloadCounter(); //reload the watchdog

while(GET_KEY_LF == 0)

{

MOTORUN();

}

//if(s_PrintParameters.m_InterFace == INTERFACE_SERIAL)

//{

//	SET_H_BUSY_S;

//}

//else

//{

//	SET_H_BUSY_P;

//}

SaveDataToFlash();

//			PrinterBusy();//设置打印机忙碌

if((count & 0x100) == 0x100)

{

count = 0;

if  (GET_LED_STA != 0)// read previous state of LED1

SET_L_LED;		  // turn LED  on

else

SET_H_LED;		  // turn LED  off

#ifdef PRINTER_AND_USB

returnstatus(0);

#endif

}

count++;

if(s_PrinterMOTOPar.m_StopPrint == ENABLE_VAR)

{

i = GetCurrentVol();

s_PrintParameters.m_CurMotoRunTime = VolRUNTime[i] - 100;//得到当前电压电机运行时间

s_PrintParameters.m_PrHeadHottime = VolTHSTB[i] - 200;//得到加热值

s_PrintParameters.m_CurMotoPWM = VolRUNPWM[i] - 100;;//得到当前电压电机PWM值

s_PrintParameters.m_CurFreeTime = VolFreeTime[i];//得到当前电压空走纸步进时间

}

}

for(count = 0;count<50;count++);

IWDG_ReloadCounter(); //reload the watchdog

for(count = 0;count<0xA00;count++);

g_LPSStatus = AD_Value[1]>>4;

if(g_LPSStatus < g_PE_Standard)

{

goto lps_feed;

}

IWDG_ReloadCounter(); //reload the watchdog

for(count = 0;count<0x150;count++);

g_LPSStatus = AD_Value[1]>>4;

if(g_LPSStatus < g_PE_Standard)

{

goto lps_feed;

}

//if(s_PrintParameters.m_InterFace == INTERFACE_SERIAL)

//{

//	SET_L_BUSY_S;

//}

//else

//{

//	SET_L_BUSY_P;

//}

//	PrinterIdle();//设置打印机空闲

#ifdef RD_TYPE_PARALLEL

#ifdef RD_PARALLEL_PE

SET_L_PE_P;

#endif

#endif

SET_L_LED;

g_PaperEnd = 0;

}

}

void FeedLabPaper(void)

{

#ifdef PRINTER_LABEL

unsigned short temp,j,count,runcount;

if(GET_KEY_LF != 0)

{

return ;

}

for(j=0;j<0x3000;j++);

if(GET_KEY_LF != 0)

{

return ;

}

for(j=0;j<0x1000;j++);

if(GET_KEY_LF != 0)

{

return ;

}

runcount = 0;

count = 3200;

while(count--)

{

ClearMoto();

IWDG_ReloadCounter(); //reload the watchdog 重装载看门狗定时器

SetMotoPWM(PrintMotoHalfStep[g_MotoAll  & 7],10);

g_MotoAll++;

Mtrig();

for(j=0;j<0x3000;j++);

FitWave();

//SendChar(AD_Value[1]>>4);

//SendChar(g_PE_Staue);

//-------------------------------处理标签位置----------------------------------------------------------

temp = g_PE_Staue^0xff;// 1

Trg = temp & (temp ^ Cont);// 2

Cont = temp;// 3

if((Trg&1) == 1)

{

runcount = s_PrintrConfig.m_HotLineTOPD +  s_PrintrConfig.m_HotLineTOOUT + (s_PrintrConfig.m_BarPaGap / 2) ;

break;

}

}

for(;runcount>0;runcount--)

{

ClearMoto();

IWDG_ReloadCounter(); //reload the watchdog 重装载看门狗定时器

SetMotoPWM(PrintMotoHalfStep[g_MotoAll  & 7],10);

g_MotoAll++;

Mtrig();

for(j=0;j<0x3000;j++);

}

ClearMoto();

#endif

}